ERDC   MSRC   PET  Program, Year 5

Meta-CTA Focused Effort Proposal

#### Focused Effort Title:	A Coupled Hydrosphere Prediction System-a Problem Solving

Environment Implementation

Organizations:	The Ohio State University

Ohio Supercomputer Center

University of Texas

Florida State University

Leads:  CTA leads at each participating institute:

K. Bedford (CWO, Ohio State University)

R. Pritchard (CCM,Training, Ohio Supercomputer Center)

M. Wheeler (EQM, University of Texas)

G. Fox (C/C, Florida State University)

Email Address:	Bedford.1@osu.edu

Telephone:	614/292-7338 (K. Bedford)  fax  614/292-3780

Statement of Work:

## Background

The Department of Defense is faced with extremely challenging problems marked by their size, scope, and immediacy.  Typically, these problems tend to fall into two major categories; those that are rationally derived through planned and executed action, and those that are a result of probabilistic natural events.  In the former category are such examples as troop landings, amphibious assaults, bio/chemical weapons dispersal, submarine warfare, and military field site recovery.  In the second class, analysis and forecasting of natural events is necessary with examples including hazardous events such as hurricanes, tornadoes, and siroccos; environmental disruptions such as spills, floods, volcanoes; long-term effects, and energy redistribution such as light, electromagnetic, and acoustic transmission through various media.

Creating accurate, timely solutions requires knowledge and manipulation of a vast array of interacting physical, biological, and chemical mechanisms and the technical problems to be addressed in order to solve the problems are among the most challenging.

- Because data are sparsely collected, databases alone are insufficient and therefore only an integrated model/data system will work;

- Models used to date are quite domain specific, typically spanning at most a two-decade resolution bandwidth, and consequently must use a wide variety of empirical procedures to specify unresolved geometric and physical activity;

- The processes to be depicted in these model/data systems are three dimensional, non linear, and contain sharp spatial and temporal gradients that can appear in any portion of the domain for long or short periods of time;

- The data and model assets required to seek these solutions are contained in a variety of  geographically and computationally diverse locations;

- The operational/military staff seeking these solutions will also be geographically dispersed, possibly of quite different skill sets, and potentially mobile; and

- The problems, being new and unique each time, often require quick solutions that must be solved in an environment that allows rapid solution method identification and implementation.

This summary contains both model and model management issues

In looking at the model issues described above, the research community has concentrated on several areas of improvements; increasing grid refinement to extend the solution resolution band width, improved solvers to allow access to more efficient algorithm deployment on parallel systems, and model coupling of domain specific codes with somewhat overlapping resolution and spectral content.  The first two areas have been traditional pursuits of researchers for years, but the coupling issues are a relatively new undertaking and result in a geometric increase in machine, algorithm, communication, software, and analysis complexity.

Simultaneously with and in response to the rapid increase in model complexity, software engineers have also been creating systems for organizing, managing, and accessing the model complexity and the resulting welter of data and decisions.  The components required to meet these model management objectives include visualization, data warehousing and mining, authoring tools, and communication and collaboration environments.  Interface tools for accessing these models and management components are also required.

The ERDC/HPCM-PET/CTAs have been at the forefront of developing the various required model and model management tools summarized above.  Over the last several years of ERDC-PET, the CTAs have made significant strides in testing all the components necessary to create a fully integrated model and model management system, especially in the areas of integrated atmospheric, surface water, and ground water flows.  All that is necessary now is a meta focused effort whose goal is to, for the first time in ERDC-PET, pull all the components together and create, implement, and test the final integrated system.

# Project Goal

The goal of this focused effort is to create and test a first version of an integrated hydrosphere prediction system through the use of modeling strategies developed in EQM and CWO and model management strategies expressed in a formal Problem Solving Environment (PSE) approach formed from tools developed in the Communication and Collaboration ( C/C )and CCM CTAs at other MSRCs.

Technical Progress to Date

In CWO, the primary focus has been to integrate circulation, wave, temperature, and sediment transport models into an organic system that operates on the same grid, space, and time scales.  In such a system, the coupling physics at the air-sea and water-bottom interfaces is crucial as is proper depiction of the two-way couplings between the code components. The system now contains nested features that allow higher resolution in near shore and shallow regions.  The primary codes unsolved are CH3D, CH3D-SED, CH3D-COSED, WAM, STWAVE, and WCBBL.  Applications have been made to the Adriatic Sea, the Atlantic Coast, and Lake Michigan.

EQM

TO ALL  CTA’s PLEASE CONSTRUCT

A THREE OR FOUR SENTENCE SUMMARY

Pertinant to this proposal, C/C has developed "computing portal" technologies, which use commodity middleware to link distributed high-performance backend resources (computers, databases, etc.) with location-independent web-based front-ends.  Between the work on the Land Management System (LMS) sponsored by ERDC, and work on the Gateway system (joint work of Syracuse/FSU and OSC, sponsored by ASC and recently ARL PET programs), the underlying technologies are nowfairly well understood and are ready for wider deployment

In CCM, a preliminary PSE has been developed and demonstrated (at SC99) by OSC in cooperation with the Information/Communication group from Syracuse.  This PSE provides the basic tools for a web browser front end for the proposed project.

# Research Plan

As seen above, all the components are sufficiently developed to allow a first test system integration.  The meta system will be tested by application to a site of DoD relevance.  While the goal of a complete hydrosphere prediction system remains, this focused effort concentrates on deploying a subset of the full suite of models sufficient to allow a full test of the PSE integrated hydrosphere prediction system.

The PSE is the system integration and management glue and the implementation and testing of a fully functioning deployment of the PSE remains the central goal of this focused effort.  The objective of the PSE is to provide a problem-oriented interface that brings high performance computing (HPC) resources to the desktop via the web.

HPC resources include supercomputer mainframes, mass storage systems, databases, workstation clusters, collaborative tools, and visualization servers.  In addition to seamless access to resources, the researcher is provided with a set of tools for composing applications from independently developed modules.  The user is assisted by a knowledge system to select adequate components.

The proposed PSE will be implemented as a three-tier system (Figure 1), a prototype of which was successfully demonstrated at SC99 within the DoD High Performance Computing Modernization Program (HPCMP) Gateway project. A web browser based front end provides visual tools to define a problem to be solved.  The middle tier implements a distributed component model (CORBA components in Java) and transforms the problem descriptor into a computational task to be executed using back-end resources.  The middle tier components are placed in a hierarchical collection of object containers (similar to the JavaBeans model).  These containers provide common services such as component life-cycle management, component persistence, security, and inter-component communications based on event notification (similarly to DCOM and Enterprise JavaBeans).  This way the task of the module developer is reduced to implementation of the module business logic only.  Generally, the modules will act as proxies for services rendered by the back-end.  Here we follow the standard industrial model of accessing databases:  a middle tier component implementing a JBDC interface serves as a proxy of the back-end DBMS.  We extend this model to include HPC resources. For example, a proxy module for job submission and control will implement the emerging Grid Interface, currently approximated by the GLOBUS meta-computing toolkit API.

Implementation of this PSE involves many challenges. Research issues to be addressed here include specification in the problem descriptor of how the modules interact with each other (within a single address space and meta-application couplings; grid-to-grid communication and boundary exchanges, and different spatial and temporal resolutions, for example). In most cases the couplings require high performance communication protocols.  A COBRA-based system, which is very adequate for communications among middle tier components, does not guarantee the performance needed for communications among computational objects on the back-end.  Our research goal here is to provide a solution in the form of proxy communication channels between proxy objects in the middle tier to be realized in the back-end as platform-dependent high performance channels.  This kind of solution is necessary to fulfill our fundamental goal of separating the process of generating the problem description in the front-end from the complete details of actual implementation of the application in the back-end.

Based upon this PSE concept overview and the existing ERDC components listed above, a multi-step research plan for this focused effort year is proposed.  The basis of the plan is to compose the simplest integrated system possible that contains all the technical features required to fully test the PSE-coupled system approach.  Once implemented and analyzed, tests for speed, scalability, operability, and domain adaptability can be undertaken during a follow-on project. This work will be closely linked to the C/C  Focused Effort proposal on computing portals, which emphasizes mechanisms to generalize the development of such portals (PSEs) and reduce the level of effort and"one-off" code required to produce a particular PSE.  This project will provide important input into the development of appropriate abstractions for portals, and can serve as a testing ground for ideas developed in the other FE’s.

With reference to Figure 1, we make the following assumption:  Some of the most time consuming portions of the full PSE implementation are the portions that use knowledge databases, user group profiles, and resource identification tools to take a user’s loosely defined problem and identify the solution approach, specific models, and specific data requirements. The remaining PSE attributes are the authoring tools, visualization and collaboration, and model/data asset components.  This latter set of tools form the core system integration components regardless of how much knowledge the user has about the models, grids, and data bases. Therefore we call these the core PSE attributes and our first year tests concentrate on implementing and evaluating these components.

Therefore our initial PSE will consist of a core PSE as based upon a core system of coupled models. The models to be coupled include the nonlinear CH3D-SED/WAM system, QUAL-II and the groundwater system.  It is assumed that the structure of the models is known and given; the simulation period and all data sets are in hand; the initial grids are available and already in a digital database.  Therefore the PSE components required include model/data asset management, collaboration, and visualization. With this basis in mind, the series of tests to be performed involve an initial series of tests with co-located collaborators, models, computers, and users.  The successive series of tests that follow involve increasing separation of the models, data, users, and analysts.

# Deliverables

| Administrative Tasks |  |  |  |
| --- | --- | --- | --- |
| ▪ ▪ ▪ ▪ | All Hands Meeting Mid Year All Hands Meeting Presentations for the PET Midyear Review and Annual Review | 4/2000 10/2000 9/2000 2/2001 |  |
| ▪ ▪ ▪ | Inputs to PET Annual Report Quarterly Progress Reports 6/2000 Final PET Technical Report | 3/2001 12/2000 3/2001 |  |
| Model Tasks |  |  |  |
| ▪ | Full coupling of CH3D-SED, WAM, WCB2L, CE-QUAL II | 7/2000 |  |
| ##### PSE Tasks |  |  |  |
| ▪ | Incorporation of full MPI, OPEN MPI model communication system | 8/2000 |  |
| ▪ | Creation of first level PSE for a given model/basin, collaboration group | 9/2000 |  |
| ▪ | Identification and implementation of collaboration visualization tools for given models, basin, and collaboration group | 9/2000 |  |
| ##### Integrated System Tasks |  |  |  |
| ▪ | First test of coupled system for fixed basin, geometry, fixed collaboration groups. Co-location for models and collaborators | 12/2000 |  |
| ▪ | First test of coupled system with remote collaborators | 1/2001 |  |
| ▪ | First test of coupled system with remote collaborators and models for fixed basin, known conditions and collaborators | 3/2001 |  |

# Customer Commitment/Involvement

Jeff Holland, ERDC, Coastal and Hydraulic Laboratory

# Required Resources

- Total Project

| 1. Personnel |  |
| --- | --- |
| 2. Travel |  |
| 3. Equipment |  |
| 4. Materials and Supplies |  |
| 5. Direct Cost Total |  |
| 6. Indirect Costs |  |
| 7. Total Costs |  |

- University Breakdown

|  | Direct | Indirect | Total Cost |
| --- | --- | --- | --- |
| 1. Ohio State University |  |  |  |
| 2. Ohio Supercomputer Center |  |  |  |
| 3. University of Texas |  |  |  |
| 4. Florida State University |  |  |  |
| 5. Total Costs |  |  |  |

- Complete University Budgets

| ### Personnel | fte. | sal. | ben. | oh | Total |
| --- | --- | --- | --- | --- | --- |
| 1. Academic Lead |  |  |  |  |  |
| 2. Academic Lead |  |  |  |  |  |
| 3. Research Scientist |  |  |  |  |  |
| 4. Graduate Students |  |  |  |  |  |
| Subtotal I: |  |  |  |  |  |
| #### II. Travel |  |  | cost. | oh | Total |
| 1. Vicksburg |  |  |  |  |  |
| 2. Scientific Meetings |  |  |  |  |  |
| Subtotal II: |  |  |  |  |  |
| - Equipment |  |  |  |  |  |
| - Materials and Supplies |  |  |  |  |  |
| Total Cost: |  |  |  |  |  |