#

#			Copyright (c) 1993 by

#			Advanced Visual Systems Inc.

#			All Rights Reserved

#

#	This software comprises unpublished confidential information of

#	Advanced Visual Systems Inc. and may not be used, copied or made

#	available to anyone, except in accordance with the license

#	under which it is furnished.

#

#	This file is under Perforce control at AVS in:

#	$Id: //depot/express/fcs992/lic/doc/licdiag.doc#1 $

#

licdiag - a wrapper for the FLEXlm licensing mechanism

licdiag is a wrapper for the FLEXlm licensing mechanism used by AVS and its

components. It is a shell script that examines your license file and license

set up, troubleshoots licensing problems, helps you start and stop the daemons,

and makes various recommendations depending on the platform.

The first thing it does is to determine your host's type; it then checks the

validity of your AVS path, and looks for a valid license file and analyzes it

for syntax errors and consistency. It will tell you if this license file

exists, if it is readable, whether it has any valid FEATURE AVS (and its

derivatives) lines, checks for SERVER lines for limited floating and node

locked licenses, and identifies evaluation licenses.

You are then prompted with a ten-item menu with the following functions:

1 - Start or restart the daemons:

o The license file should contain exactly one 'DAEMON avs_lmd' line

o The license file should contain at least one SERVER line

o The machine's id should be listed in one of the SERVER lines

o No other lmgrd/avs_lmd processes should be running; they are killed

before proceeding

o All lock files are removed

o Make sure the port specified in the SERVER line is not being used by

another application

o After starting the daemons, scan the log file for error messages

2 - Stop the daemons: Similar as above

3 - Get status:

o Determine whether any daemons are running

o Execute lmstat

o Verify ability to write and lock files in /usr/tmp

4 - Show lmgrd version: run lmgrd -version

5 - Reread license file: similar to #1

6 - Check networking: makes recommendations on network troubleshooting

7 - Help on starting at bootup: makes recommendations depending on the platform

and OS revision

8 - Show license file: the license file under scrutiny

9 - Show machine's hostid: both as it should be shown in the license file in

the case of node locked license, and the string to be used for obtaining

licenses

0 - Show the current log file