DBTOOL - A Database Administration tool for Oracle on MPP Systems

=================================================================

TABLE OF CONTENTS

-----------------

1. Introduction

2. Architecture Overview

3. Group Configuration Management

4. Environment Setup

5. Usage Description

APPENDIX

A. Example Group Configuration File

1. Introduction

DBTool addresses the needs of an OPS (Oracle Parallel Server) user

in context of a system architecture that supports parallel processing

on a large number of processing nodes. Its objective is to provide

with a single-point control to all those operations that generally

involve an enormous amount of repetitive work and take away not only

the user's ease but, to a great extent, the scalability too.

DBTool has a notion of 'instance-grouping'. An 'instance-group' is

defined as a logical entity that encapsulates in it the definitions

of a set of OPS instances. The reference to an instance-group enables

handling of operations (of similar nature) on multiple OPS instances

through a single user-request. For details on the various operations,

see the "Usage Description" section of this document.

2. Architecture Overview

The elements listed below collectively define the architecture of

DBTool:

o A globally accessible repository :

oo CCR (Centralized Configuration Repository):

This is a data storage element (typically a VSD, for SPx)

that is equally accessible to all the nodes of the system.

It is used to store the mapping between the name of each

instance-group and all of its configuration details as

specified by the user at the time of registration.

o A set of processes, namely :

oo DBTool Instance:

This is the process that gets initiated when user invokes

DBTool. Its role mainly consists of parsing and validation of

user input, interaction with concerned DBTagent(s), handling

of CCR , and output of results to user. A DBTool instance can

be brought up from any configured node of the system.

oo DBTagent Instances:

DBTagent is a daemon running on each node.

It executes (on the local node) each of the operations

requested by the DBTool instance and responds about its

results.

3. Group Configuration Management

The request for each DBTool operation needs a reference to an

instance-group. A specific DBTool operation can either be performed

for all of the OPS instances addressed by the instance-group, or on a

specific subset of OPS instance(s) of the instance-group.

As described earlier, an instance-group is a collection of multiple

instance-definitions. An instance-definition is constituted of the

values of a set of elements (termed as 'configuration elements') that

are essential to uniquely identity the OPS instance within its

instance-group and to support various operations (for example startup

and shutdown) of the OPS instance. The configuration elements

identified thus far are :

INSTANCE_ID : Uniquely identifies an instance within the group

NODE_NAME : Name of the node on which the instance would run

ORACLE_HOME : Value of ORACLE_HOME for the instance

ORACLE_SID : Value of ORACLE_SID for the instance

PARAMETER_FILE : Parameter file for the instance (optional)

DBTool requires the registration of an instance_group into a

Central Configuration Repository (CCR) before most operations. The

registration into the CCR is made through a 'group-definition file'.

The group-definition file is a simple, user-editable text file that

is divided into two logical sections:

o Global Information Section :

It contains (optionally) the default values for specific

configuration elements. If, for an OPS instance, the value of a

configuration element is not specified in the Individual

Configuration Section, its value is taken from the Default Global

Section.

o Individual Configuration Section :

It contains the values of the configuration elements separately for

the instances of the group.  The configuration element INSTANCE_ID

is used to uniquely identify an instance within a group. The

configuration elements that are mandatory in this section for a

specific instance-definition are: INSTANCE_ID, NODE_NAME and

ORACLE_SID. The configuration element ORACLE_HOME, if not specified

in the Global Information Section, must appear in this section.

However, if ORACLE_HOME is already specified in the Global

Information Section, it can either be accepted as it is for

the specific instance-definition by skipping it in the Individual

Configuration Section, or it could be overridden by mentioning it

in this section.

[ Note: A sample group-definition file is provided in the Appendix ]

4. Environment Setup

Certain environment variables need to be set up before DBTOOL can

be used.

o DBTCCR	name of the device that will be the Centralized

Configuration Repository. The device must be a

block special VSD (e.g. /dev/vsdccr). This VSD

must also be created before using DBTOOL.

o LKMGR_CONF	DBTOOL can be used for starting or shutting down

Lock Managers on all the nodes desired, or for

starting or shutting down DBTOOL agents on the nodes.

Before these operations can be used, a lock manager

configuration file must be set up. This environment

variable names this configuration file.

The /etc/services file must be updated on each node to add an entry

that would specify the port for the tcp/ip protocol-based service of

'dbtagent'. The entry may look like (port number may vary):

dbtagent 	1877/tcp

A VSD must be created to serve as the CCR (Centralized Configuration

Repository). This is the device into which the configuration of all

the groups would be stored. The handling of CCR is transparent to the

user. The user simply needs to create it and make its presence

known through the DBTCCR environment variable.

After the above setup is done, run DBTOOL to start the DBTOOL agents

on all the configured nodes. This can be done by:

dbtool -dbtstart

5. Usage Description

NAME

dbtool - Database Administration Tool for Oracle Parallel Server

SYNOPSIS

dbtool -INITCCR

dbtool -SHOWCFG [ grouplist ]

dbtool -REGGRP groupname group_definition_file [-UPDATE]

dbtool -DELGRP grouplist

dbtool -ORASTART groupname [-OPTS :options:]

[-INST instancelist | -NODE nodelist]

dbtool -ORASHUT groupname [-OPTS :options:]

[-INST instancelist | -NODE nodelist]

dbtool -GENREDO -SCRIPT file [-MEMBERS file] [-ST start_thread]

-NT num_threads -SG start_group

[-GPT groups_per_thread] [-MPG members_per_group]

[-SZ size] -PRE prefix [-PVT]

dbtool -CRTREDO groupname {-INST instance_id | -NODE nodename}

[-ST start_thread] -NT num_threads -SG start_group

[-GPT groups_per_thread] [-MPG members_per_group]

[-SZ size] -PRE prefix [-PVT]

dbtool -LMSTART [-OPTS :options:]

dbtool -LMSHUT [-OPTS :options:]

dbtool -DBTSTART [-OPTS :options:]

dbtool -DBTSHUT [-OPTS :options:]

grouplist ::= groupname [{,groupname}...]

instancelist ::= instance_id [{,instance_id}...]

nodelist ::= nodename [{,nodename}...]

DESCRIPTION

The following options are available for dbtool :

-INITCCR  Initialize the Centralized Configuration Repository. This

is automatically done when dbtool operations are performed.

The option is provided, however, so that it can be used for

re-initialization in situations when the CCR is been

initialized but is yet unusable due to corruption.

-SHOWCFG  Display the configuration of either a specific set of

groups, or all the groups defined as yet. If no groupname is

specified, configuration of all the existing groups would

be displayed.

-REGGRP   Register the definition of a group (named as <groupname>)

into the centralized configuration repository. The details

about the configuration of the group are input through a

text file (named as <group_definition_file>)

-UPDATE   When used in conjunction with -REGGRP it specifies that if

a group with the specified <groupname> already exists, its

existing definition is allowed to be replaced by a new

definition as per the contents of the specified

<group_definition_file>.

-DELGRP   Eliminate the definition for the specified group(s) from

the central configuration repository.

-ORASTART Startup either, all of the OPS instances of the specified

group (named as <groupname>) [when -INST option is not used]

or, a selective set of OPS instances (identified through

the use of -INST option) belonging to the specified group

(named as <groupname>)

-INST	  This option cannot co-exist with -NODE. When used in

conjunction with -ORASTART or -ORASHUT, it identifies the

specific OPS instances to be started up or shut down. The

<instance_id>'s used with this option correspond to those

mentioned within the group_definition file used for the

registration of the concerned group (named as <groupname>).

-NODE	  This option cannot co-exist with -INST. When used in

conjunction with -ORASTART or -ORASHUT, it identifies the

specific OPS instances to be started up or shut down. Each

OPS instance is identified by the corresponding node's name

on which it is supposed to run.

-OPTS     A way of specifying extra options related to the task that

will be carried out. These options must be enclosed in a

pair of ':' characters.

When used in conjunction with -ORASTART, it specifies the

desired special startup options (as FORCE, RESTRICT, OPEN,

RECOVER, MOUNT etc.) as used with command line mode of

SQL*DBA. When used in conjunction with -ORASHUT, it specifies

the desired special shutdown options (as ABORT, IMMEDIATE or

NORMAL) as used with command line mode of SQL*DBA.

With -LMSTART, it specifies the desired special options as

used with the lock manager.

-ORASHUT  Shutdown either, all of the instances of the specified group

(named as <groupname>) [when -INST option is not used]

or, a selective set of instances (identified through the

use of -INST option) belonging to the specified group (named

as <groupname>)

-GENREDO  Generate a SQL script for creation of redo threads.

-SCRIPT     specifies the file where the script is written

-MEMBERS    specifies the file where the list of all redo

log members is written. The member list can be

used, for example, to "prepare" the space for

the members at the OS level.

-ST 	      specifies the thread number to start with

(default 2).

-NT 	      specifies the number of threads.

-SG 	      specifies the group number to start with.

-GPT 	      specifies the number of groups per thread

(default 3).

-MPG        specifies the number of members in each group

(default 1).

-SZ 	      specifies the size of each redo log member.

This can be a decimal integer optionally followed

by 'M' or 'K' (default 500K).

-PRE        specifies the prefix to be used in generating

the redo log member pathnames. The prefix will

be suffixed with

<thread number>_<member number within thread>.

-PVT 	      optionally specifies that the threads are to be

enabled as PRIVATE (default PUBLIC).

For example, if prefix is "/dev/vsd.x", the member names

generated for thread 5 may look like

"/dev/vsd.x5_1", "/dev/vsd.x5_2" etc..

-CRTREDO  Generate and execute a SQL script for creation of redo

threads. An instance needs to be specified where the script

will be executed.

-INST       specifies the instance where the script will

be executed. It may be specified by the use of

the use of the -INST or the -NODE option.

-ST 	      specifies the thread number to start with

(default 2).

-NT 	      specifies the number of threads.

-SG 	      specifies the group number to start with.

-GPT 	      specifies the number of groups per thread

(default 3).

-MPG        specifies the number of members in each group

(default 1).

-SZ 	      specifies the size of each redo log member.

This can be a decimal integer optionally followed

by 'M' or 'K' (default 500K).

-PRE        specifies the prefix to be used in generating

the redo log member pathnames. The prefix will

be suffixed with

<thread number>_<member number within thread>.

-PVT 	      optionally specifies that the threads are to be

enabled as PRIVATE (default PUBLIC).

For example, if prefix is "/dev/vsd.x", the member names

generated for thread 5 may look like

"/dev/vsd.x5_1", "/dev/vsd.x5_2" etc..

-LMSTART  Start the lock managers on all configured nodes. The

configuration information is obtained from the file

whose value is given by the LKMGR_CONF environment variable.

Options provided with -OPTS will be used as options to

the lock managers.

-LMSHUT   Shut down the lock managers on all the configured nodes.

-DBTSTART Start the DBTOOL agents on all configured nodes. The

configuration information is obtained from the file

whose value is given by the LKMGR_CONF environment variable.

When an action is requested by running dbtool, messages are

sent to the respective DBTOOL agents on the desired nodes

and the action is performed by the DBTOOL agents.

-DBTSHUT  Shut down the DBTOOL agents on all the configured nodes.

ENVIRONMENT VARIABLES

DBTCCR		The pathname of the Central Configuration Repository

file

LKMGR_CONF	Configuration file needed by the options to start/shut

Lock Managers and Dbtool Agents

NOTES

Use of INSTANCE_ID for unique identification of an OPS instance within

its instance_group might get dropped in future. The node_name, itself,

should suffice for this uniqueness.

The Dbtool Agent writes log information in the /tmp/dbtagentlog file.

The ORACLE_HOME variable must be the same on all the nodes for the

-LMSTART and -DBTSTART options to work correctly.

CAVEATS

The options given on the command line must be in the same order

as shown in the SYNOPSIS

The Centralized Configuration Repository (CCR) must be on a block

device VSD. Use caution when running dbtool from many different

nodes at the same time. Run on only 1 node at a time, flush buffers

(using the command sync;sync;sync) before using it on a different node.

This limitation is temporary. In the future, a character device VSD

will be used which will eliminate data integrity problems.

APPENDIX

A. Example Group Configuration File

################################################################################

#                                                                              #

# Important Notes:                                                             #

#     1. Any line starting with a '#' is  a comment line.                      #

#     2. The portion of file preceeding an INSTANCE_ID configuration element   #

#        constitutes  the 'Global Information Section'.                        #

#     3. The portion of file spanning from the first INSTANCE_ID configuration #

#        element till the end of file constitutes the 'Individual Configuration#

#        Section'.                                                             #

#     4. The value of INSTANCE_ID must start with 1 and be in ascending order, #

#        in increments of 1.                                                   #

#     5. Read "Group Configuration Management" section in the documentation    #

#        for details

################################################################################

#                                                                              #

#------------------------------------------------------------------------------#

# Global Information Section                                                   #

#------------------------------------------------------------------------------#

#                                                                              #

ORACLE_HOME=/dbtool/oh

#                                                                              #

#------------------------------------------------------------------------------#

# Individual Configuration Section                                             #

#------------------------------------------------------------------------------#

#                                                                              #

# All instances use the default ORACLE_HOME specified in the Global Information#

# section

#

# The following instance does not specify a parameter file. When the instance  #

# is started, the default $ORACLE_HOME/dbs/init<sid>.ora will be used          #

#

INSTANCE_ID=1

NODE_NAME=maple

ORACLE_SID=dbt1

#                                                                              #

INSTANCE_ID=2

NODE_NAME=banyan

ORACLE_SID=dbt2

#                                                                              #

INSTANCE_ID=3

NODE_NAME=joshua

ORACLE_SID=dbt3

PARAMETER_FILE=/dbtool/oh/dbs/initdbt3.ora

#                                                                              #

INSTANCE_ID=4

NODE_NAME=sycamore

ORACLE_SID=dbt4

PARAMETER_FILE=/dbtool/oh/dbs/initdbt4.ora

#                                                                              #

INSTANCE_ID=5

NODE_NAME=pine

ORACLE_SID=dbt5

PARAMETER_FILE=/dbtool/oh/dbs/initdbt5.ora

#

INSTANCE_ID=6

NODE_NAME=palm

ORACLE_SID=dbt6

PARAMETER_FILE=/dbtool/oh/dbs/initdbt6.ora

################################################################################