# 1 Terrain Surface Model

There are many kinds of terrain modeling applications, such as flight simulators, ground vehicle simulators, and geographical information systems. Design of those applications involves finding satisfactory solutions of the following problems, typical for this type of software:

- Fast rendering of the terrain model,

- Fast transmission of the elevation data,

- Good visual quality of the rendered terrain.

It is worth stating that there is a contradiction between the first two goals and the third one. Fast rendering requires relatively small number of polygons to be displayed real-time, however the smaller number of polygons, the worse visual quality and accuracy of the terrain model. Similarly, fast network transmission of terrain data is possible only for highly compressed elevation grids. The most sophisticated compression algorithm there is published nowadays, wavelet compression, provides excellent compression ratio while the most important features of the terrain data is preserved, however for nearly-loseless reconstruction, decompression time grows substantially. Very fast decompression results in visible terrain data loss.

In the following subsections we will describe constructing terrain models from digital elevation data and satellite imagery and/or maps, focusing on optimization techniques we developed to reduce network bandwidth and improve rendering speed.

## 2 Elevation Data

Digital Elevation Models (DEM) consists of a sampled array of elevations for a number of ground positions at regularly spaced intervals. United States Geological Survey made available Digital Elevation Models for most of North American continent in several spatial resolutions, including 7.5-, 15-, 30-minute, and 1-degree. In our system, we decided to use 1-degree DEMs. The reasons for this decision include compatibility between DEMs and satellite imagery/maps accessible on the marker, and availability of the data. 1-degree DEMs for continental U.S. are freely available from http://edcwww.cr.usgs.gov/glis/hyper/guide/1_dgr_demfig/index1m.html.

The main characteristics of a 1-degree DEM are the following:

- It consists of a regular array of elevations referenced horizontally on the geographic (latitude/longitude) coordinate system of the World Geodetic System 1972 datum (WGS 72) or the World Geodetic System of 1984 (WSG 84).

- Spacing of the elevations along each longitude profile is 3 arc-seconds, what roughly corresponds to 100 meters for continental U.S. Along latitude, the same spacing is used for data covering the area south of 50 N, while north of 50 N and south of 70 N the spacing is 6 arc-seconds.

More detailed description of the USGS DEM data files can be found in [1].

## 3 Triangulated Irregular Network

Since almost all graphics hardware uses polygons as the fundamental building blocks for object description, it seems natural to represent the terrain as a mesh of polygonal elements. The raw elevation data can be trivially converted to polygons by placing edges between each pair of neighboring samples. However, for areas of any significant size, rendering the full model is prohibitively expensive. For example, a single 1-degree DEM data file, which covers the area of roughly 70 by 70 miles (120 by 120 km), consists of 1201*1201=1,442,401 elevation samples, which results in almost 3 millions triangles to be rendered.

What is more important, details of the full model are highly redundant when it is viewed from the distance, and rendering them in such cases is unnecessary and wasteful. Quite often terrain features large, nearly planar regions, which are well approximated by large polygons instead of hundreds of smaller ones. Ideally, we would like to render models of arbitrary terrain with just enough detail for visual accuracy. Additionally, in systems, which are highly constrained, we would like to use a less detailed model in order to conserve memory, network bandwidth, and maintain good rendering speed.

The aforementioned factors let us draw a conclusion that reduction of the number of polygons used to represent terrain is both necessary and beneficial for terrain rendering applications. That is why we introduced Triangulated Irregular Network (TIN) to represent a surface as a set of contiguous non-overlapping triangles whose vertices are located on the surface. We will discuss the methods we deployed to create TINs: elimination of points from full elevation sample matrix (mesh simplification), and creation of polygons—most often triangles—from the set of designated elevation points (triangulation).

### 4 Mesh simplification

The key to good quality terrain rendering is the choice of a good point importance measure. “Important” points should remain in the elevation samples set that is used to create TIN, while those, elimination of which does not reduce accuracy of the final model, can be removed.

It is obvious that removal of every n-th point from the grid will not lead to preservation of terrain shape and good visual results.

We decided to use importance measures based on preservation of the terrain curvature. We tested two such measures:

- Cross product based,

- Second derivative based.

The first algorithm is quite simple, yet provides visually very good results. The algorithm in one-dimensional case can be described as follows:

- i=1; prev=0;

- while i<number of samples do

- examine the area of the triangle build by the vertices i-th, i+1-th and prev-th (i.e. the length of the cross-product vector of vectors [prev,i] and [prev,i+1])

- if the area is smaller than threshold remove the i-th sample; else prev:=i

- increase i

The illustration of this algorithm is shown in Figure 1. Applying the algorithm in x, y, and xy directions extends it to two-dimensional space.

The drawback of this algorithm is that it has a tendency to create slopes in bowl-shaped terrain (e.g. river valleys), as shown in Figure 2. A workaround is to apply the algorithm in two directions and remove the points marked for removal during both passes.

Figure 1. Cross-product based mesh simplification (one-dimensional case).

Figure 2. The cross-product based algorithm applied to valley-type of terrain.

Second derivative based importance measures proved to be not as useful in mesh simplification. The reason was that this measure is oversensitive to high frequency variations, while important changes of slow but consistent character tend to be removed. Illustration of that is shown in Figure 3.

Examples of the actual results of mesh simplification routine using both measures are shown in Figure 4.

Figure 3. Anomalies introduced by second derivative based importance measure.

(a)

(b)

Figure 4. TINs simplified by two importance measures: (a) cross-product based,
(b) second derivative based

We decided to use in 3D GIS the cross-product based algorithm, mainly because of better visual quality of the TINs produced by it. We tested it with different values of the threshold parameter. It turned out that the optimal values of this parameter for the highest level of detail representation of terrain chunk were 0.2-0.3 (see Section 14 Terrain Scene Graph, for description of level of detail representations). These values of the mesh simplification threshold lead to tremendous gains in rendering speed, resulting from reduction of number of triangles in the terrain model.

Figure 5. Number of triangles in TIN as a function of mesh simplification threshold.

Figure 6. Total time needed for rendering of the terrain model
as a function of mesh simplification threshold.

Figure 6 shows total time needed for rendering a terrain model, including mesh simplification time, Delaunay triangulation time, and internal processing of the triangle set by Open Inventor scene rendering mechanism. The reduction factor for the terrain model obtained by applying the mesh simplification threshold equal to 0.25 was 40, when compared to time needed to render the full regular mesh of 128*1024 triangles. This ratio is even better for less detailed representations of the terrain, reaching values of hundreds.

### 5 Triangulation

A problem of subdividing a complex domain into a disjoint collection of ‘simple’ objects is referred to as triangulation. In the most common case, triangulation is a partition of the two dimensional plane into a set of triangles. The most popular type of triangulation is Delaunay triangulation, consisting of only ‘good,’ not ‘thin,’ triangles. The expression that is closely related to Delaunay triangulation, and, in fact, being the starting point for Delaunay triangulation is Voronoi diagram.

Figure 7. Voronoi polygon

As we can see in Figure 7 a single Voronoi polygon is defined by the lines that bisect the lines between a center point and its surrounding points. The bisecting lines and the connection lines are perpendicular to each other. When we use this rule for every point in the area, the area will be completely covered by adjacent polygons. The Voronoi diagram for a set of random distributed points is shown in Figure 8. Notice that the polygons on the boundary of the area are “open” because they have no neighboring points in that direction.

Figure 8. Voronoi diagram

If we use the Voronoi diagram as a basis, we can construct the Delaunay triangulation by drawing the lines between the points in adjacent polygons (see Figure 9).

Figure 9. Delaunay triangulation and Voronoi diagram are dual to each other

The main rule for the Delaunay triangulation can be formulated as the circle criterion: a Delaunay triangulation in two dimensions consists of non-overlapping triangles, such that the circumcircle of any triangle in the triangulation is empty (contains no points from the mesh). Because of that feature, Delaunay triangulations contain “even” triangles, which are particularly important for good visual quality of terrain in terrain rendering applications.

Straightforward algorithm for building Delaunay triangulation can be titled ‘Incremental Update.’ First, we take three points already satisfying the circle criterion (for example three points located in infinity), and connect them to obtain a triangle. This is our initial Delaunay triangulation. Then, one by one we add points from the set to be triangulated. The point will be added in the middle of an already existing triangle, so we have to add new three edges, as shown in . However, the resulting triangulation may no longer fulfill the circle criterion. In such a case swapping one or more edges of the triangulation will restore it to a Delaunay one ().

Figure 10. Insertion of a point

Figure 11. Swapping edges

More formal analysis of this algorithm can be found in [4].

However, there are much more efficient algorithms that may be deployed to create Delaunay triangulations. The one we used in 3D GIS works according to devide et impera paradigm:

- The point set is first divided into two sub-sets with equal number of points. Division continues recursively until each data set consists of a maximum of four points.

- Delaunay triangulations are made at the lower level, and the merging of these data sets starts. In the recursive process each region is merged together until the whole area is completed (see Figure 12).

Figure 12. Merging of two data sets.

# 6 Satellite Data

3D GIS displays satellite imagery as textures superimposed on the elevation model. We produced satellite images covering most of New York State from data coming from the Landsat 5 satellite. Its Thermappic Mapper (Landsat TM) technology produces seven separate spectral bands. Landsat TM is a second-generation earth resources satellite that combines reasonable spatial resolution (30 by 30 meters) with a reasonable range of spectral bands (the bands are of visible and near, short and mid infrared wavelengths). The first band covers the range from 450nm to 520nm, which roughly corresponds to blue light in the visible spectrum. Landsat TM data is readily available for most of the world.

We used special software, ER Mapper, to process Landsat TM bands and combine them to obtain color images. Some of the algorithms supplied with ER Mapper for Landsat TM data we used were:

- RGB_321.
This algorithm is a ‘natural’ color scene comprising TM bands 3-2-1 (approximately equal to red-green-blue visible light) imaged in the red-green-blue computer monitor guns respectively.
The composite is not totally natural because the TM bands do not exactly match the red-green and blue spectral regions; bands 1 (blue visible) and 3 (red visible) have a lower spectral range than that which the eye recognizes as blue and red. Images produced by this algorithm usually look partly realistic but sometimes there might be an apparent absence of green vegetation.

- RGB_341.
The ‘greenness’ of the natural image can be improved by substituting TM band 4 for band 2. In band 4 (centered at 0.83 um) in the near infrared, vegetation has very high reflectance. The resultant image will be a false-color one, because the response from a spectral band that is not from the visible range (has no ‘color’), into a visible wavelength so that we can ‘see’ a wavelength not normally used.

Description of other algorithms we used may be found in ER Mapper Applications [3].

# 7 GIS Database

census data; structures of tables; deepak

# 8 Wavelet Compression

We deployed wavelet compression for both textures (satellite images) and for elevation data. Detailed description of wavelet compression we implemented can be found in . In this section we will focus on the results of using this technology in 3D GIS, as far as both network transmission time and quality is concerned.

## 9 DEM Compression

We performed several tests to illustrate the way wavelet compression affects terrain shape models. We chose a typical terrain grid size 3D GIS displays, 256 by 256 samples, i.e. 25.6 by 25.6 km and the compression ratio of 32, which is significantly high yet yields almost no visible artifacts.

The chart below represents the number of triangles produced by our mesh simplification routine from two sets of data: uncompressed and compressed.

Figure 13. Number of triangles produced by the mesh simplification routine
for compressed and raw elevation data

It is easy to explain smaller number of triangles for compressed data having in mind characteristics of wavelet compression. Wavelet compressed images tend to be smoother, have less high-frequency components. That is why a mesh simplification routine based on curvature preservation is able to remove more mesh points from a compressed dataset, resulting in a smaller number of triangles.

In Figure 14 several 3D viewer windows displaying terrain models of the same area are shown. The terrain in the images has been constructed from compressed (left column) and raw data (right column). Mesh simplification with three different threshold values: 0.01, 0.1, and 0.3 has been applied. What is worth noticing, terrain shape deformations introduced by wavelet compression become more evident for bigger values of mesh simplification threshold, while being negligible small values.

(a)

(b)

(c)

Figure 14. Visual comparison of terrain models generatedfrom compressed (left column) and raw (right column)
elevation data for different values of simplification threshold: a) t=0.01, b) t=0.1, c) t=0.3

## 10 Texture Compression

One of the characteristics of wavelet compression is dependency of compression effectiveness on features of individual images being compressed. Usually, the more “detailed” image (with high-frequency components), the lower compression ratio can be gained for fixed image quality. Restored image quality improves as the number of passes of wavelet compression algorithm used to decompress image grows. 3D GIS for satellite images runs usually six to seven passes of wavelet decompression algorithm. This yields compression ratios between 10 and 36 for 7 passes, and 28 to 75 for 6 passes. Each pass adds more information to the compressed stream, hence decreases compression ratio. This wide variety of compression ratios for the same number of the wavelet compression algorithm passes is an illustration of how the final compression gains are dependent on individual image.

Decompression time for sample satellite images of size 512 by 512 pixels averages to 1.88 seconds for 6 passes, and to 2.14 seconds for 7 passes. The machine used to take these measurements was SGI Indigo 2 with MIPS R10000 processor, CPU clock speed at 195 MHz.

## 11 CPU versus Network Bandwidth

As was shown in the previous section, wavelet compression can be quite expensive in terms of time necessary for decompression. Therefore, for machines with relatively slow CPU and fast network connection it may be profitable to give up wavelet compression and download larger file at a gain of reduced terrain model preparation time.

We define a critical point as a combination of CPU power and network bandwidth of a particular system for which decompression time is as long as time necessary to download the raw, uncompressed file. For example, during our tests for SGI Indigo w/R10000 and network connection at 2 Mbps both times were roughly the same.

In Figure 15, a chart with critical points for three systems on which we have tested our decompression routine is shown. As we can see, wavelet compression introduces speedups of total time necessary for network transmission and decompression for all types of low-end network connections (modems up to 56k, and ISDN). It is however faster to transfer uncompressed data through cable modem or Ethernet (1Mbps) to slower machines, such as PCs with older Pentium processors or low-end SGI workstations.

Figure 15. Critical points for SGI Indigo, SGI Indy, and PC

# 12 Implementation

## 13 Choice of 3D Graphics Toolkit

We considered multiple options when we were choosing programming tools to be used in the 3D GIS viewer. One of the alternatives, the simplest, was constructing virtual worlds in Virtual Reality Modeling Language (VRML) and employment of standard VRML browsers to display them. This solution is very simple, but has important disadvantages:

- Files describing terrain shape created in pure VRML are huge (size of hundreds of megabytes) and therefore require enormous disk space and cannot be sent in that form over the network;

- Lack of interaction capabilities. One of our essential design goals was providing users with a chance to choose the information to be displayed, such as maps, satellite images, or weather information. VRML 1.0 provided no means of implementation of almost any interaction with the user;

Another possible solution was employing VRML 2.0 as a language to code virtual worlds. In this case special nodes for particular data types and special scripting nodes in Java could have been introduced. This approach could solve the problem of huge elevation data files, which could have been uncompressed by Java scripts. The cost of Java programs is still in their robustness—native programs for the same platform are still much faster then their Java-implemented equivalents. This solution still does not entirely solve the problem of interaction with the user.

The third possibility was to develop custom browser tailored especially for use in GIS, implemented either in C or C++, using one of the popular 3D modeling libraries, such as OpenGL or Open Inventor.

In such a custom browser, the problem of large data files can be easily solved by introduction of special objects or procedures that would be used to handle compressed data and display it appropriately in the browser.

Interaction with the user can be very easily implemented, with constraints of only programming abilities of people who implement the system. The browser could be designed flexible enough to allow the user control the shape, appearance, and behavior of scene elements in almost any imaginable way.

The possibility we abandoned at the very beginning of design stage was writing the native Motif/OpenGL application. The complexity of those programming environments and lack of high-level components when compared to Open Inventor were much more important than the profits we would have gained as far as performance is concerned.

The tool that seemed perfect when we were making the decision about a choice of the programming environment, was Open Inventor 2.1. Main advantages of using Open Inventor, among many others, are:

- Portability. Open Inventor is available not only for SGI platform, but also on several other Unix systems, and also on PC. As long as platform specific system functions are not used the same Open Inventor code can be successfully compiled and run on a wide range of hardware/software platforms.

- Variety of different user interaction features. Inventor provides hundreds of building blocks, which can be used to interact with the user, such as sensors, manipulators, draggers, and property editors.

- Compatibility with VRML 1.0. Open Inventor accepts all models in VRML format, and displays them in its standard generic viewers without any need of modifications by a programmer. VRML geometry may be incorporated with the Open Inventor models on the fly.

- Open Inventor is written in pure ANSI C++. Power and flexibility of this language convince any programmer to choose it as a programming language when performance of the software is crucial for project’s success.

## 14 Terrain Scene Graph

Open Inventor manages all of the objects defining a 3D scene in a 3D scene database, which stores shape, property, group, engine, and sensor objects, used to create a hierarchical 3D scene.

The basic building block used to create three-dimensional scene databases in Inventor is called a node. Each node holds a piece of information, such as a surface material, shape description, geometric transformation, light, or camera. All 3D shapes, attributes, cameras, and light sources present in a scene are represented as nodes.

An ordered collection of nodes is referred to as a scene graph. The scene graph is stored in the Open Inventor database. Open Inventor takes care of storing and managing the scene graph in the database. The database can contain more than one scene graph.
After a scene graph is constructed, a number of operations can be applied to it, including rendering, picking, searching, computing a bounding box, and writing to a file.

Classes of database primitives include shape nodes (for example, sphere, cube, cylinder, quad mesh), property nodes (for example, material, lighting model, textures, environment), and group nodes (for example, separator, level-of-detail, and switch). Other special database primitives are engines and sensors. Engines are objects that can be connected to other objects in the scene graph and used to animate parts of the scene or constrain certain parts of the scene in relation to other parts. A sensor is an object that detects when something in the database changes and calls a function supplied by the application. Sensors can respond to specified timing requirements (for example, “Do this every 5 seconds”) or to changes in the scene graph data.

Terrain model in 3D GIS is divided into chunks, or quadrants of size corresponding to one satellite image, or more generally, one image used as a texture. Typically, the size of one chunk is ¼ by ¼ of latitude/longitude degree, what is roughly 25.6 by 25.6 km.

A terrain chunk is represented in 3D GIS as a special Open Inventor grouping object, SoSeparator with two main children: the texture used for this terrain chunk (SoTexture2) and the terrain geometry, stored in SoLevelOfDetail node. SoLevelOfDetail was designed to specify the appearance of the same object with varying levels of detail. The children of this node are arranged from highest to lowest level of detail. The size of the objects when projected into the viewing window determines which child to use. This node is ideal for deployment in applications requiring the fastest rendering possible. To determine which child should be rendered, Open Inventor computes the 3D bounding box of all children in the level-of-detail group. The size of the projected on the viewport bounding box is compared with the screenArea field of the SoLevelOfDetail, which is an array of floats.

Deployment of this class in 3D GIS allowed us gain substantial improvements of rendering speed by preparing for each terrain chunk its three representations: the most detailed one, less detailed, and the most coarse one. The number of triangles decreases dramatically in each representation, what yields much faster redisplay.

## 15 Terrain Scene Description File Format

Although the GIS system is shipped with its own Graphical User Interface (GUI), it is possible to use it as a specialized virtual reality viewer. The URL for the file to be displayed can be easily entered in the Open Location Dialog window. The GIS viewer accepts all standard VRML 1.0 and Open Inventor 2.1 files. Apart from it, we introduced several language extensions, specific to our viewer, yet still compliant with Open Inventor interchange file format. Those extensions are mainly based on SoInfo node, which function in standard Open Inventor files is only to provide some comments to humans. However, after retrieval of Scene Description File (SDF) from the HTTP server, when the Open Inventor database manager, SoDB, has read the content of SDF, and parsed it to create a scene graph, an instance of TagParser object is created to find all registered GIS tags in the scene graph, and call associated with them callback functions.

A special class, SceneBuilder, was introduced to create Open Inventor object hierarchy corresponding to SDF. Objects of this class are employed when the user specifies via GUI what terrain areas he/she would like to visit. A scene graph created by SceneBuilders is no more than a series of GIS tags, passed then to the TagParser object.

We defined the following GIS tags:

- GIS_Root provides the main part of all the URLs used for downloading terrain data: both textures and elevation samples. The string field of this SoInfo node should contain a valid URL, for example “http://sandman.npac.syr.edu:8888/GIS.” Note that the string does not include the trailing slash. The location of textures and elevation files are created by appending “/Textures” or “/DEM” to the value of GIS_Root. Presence of more than one tags of this kind is treated as a SDF format error.

- GIS_Origin informs what geographical coordinates (latitude, longitude, and altitude) correspond to Open Inventor’s (0,0,0) point. The valid format of the string field of this SoInfo node is “%d:%d:%d,%d:%d:%d,%d,” where %d stands for an integer. The first triplet is used to calculate the value of the origin’s longitude, while the second triplet corresponds to its latitude. The last number is the origin’s altitude in meters. Presence of more than one tags of this kind is treated as a SDF format error.

- GIS_Bounds is used in code controlling GIS viewer’s camera position. If a GIS_Bounds tag has been found in the SDF, a viewer’s camera position will be constrained to the inside of the 3D box defined by this tag. The tag consists of two geographical coordinates of the extents of the terrain being rendered. The first pair of coordinates determines the northwest corner of the box inside of which a viewer’s camera is allowed to move. The other set of coordinates is used to find the south and east boundaries of this 3D area. The valid format of the string field is “%d:%d:%d,%d:%d:%d %d:%d:%d,%d:%d:%d.” This tag is optional—if not supplied, the viewer’s camera position will not be constrained. However, presence of more than one tags of this kind is treated as a SDF format error.

- GIS_Scale is used to determine the scaling factor when transforming geographical coordinates to Open Inventor’s coordinate system. The valid format of string filed is “%d %d %d,” where the first integer describes how many Open Inventor’s units corresponds to one degree of longitude; similarly, the second one plays the same role as far as latitude is concerned. Finally, the third number is the divider used to obtain Open Inventor’s y-coordinate for a particular altitude. Presence of more than one tags of this kind is treated as a SDF format error.

- GIS_Texture provides information about one of possible texture layers. The format of the string field should be “%s,%s,” where %s stands for a string of characters acceptable in the standard UNIX path. The first part of this field (up to the comma) is appended to the value of GIS_Root concatenated with “/Textures/.” The resulting string should point to the valid directory of the HTTP server. Requests for image files from subdirectories of this directory will be sent to the server.

- GIS_RectArea and GIS_CircleArea are tags that introduce “hot spots” to the terrain model. Those areas have an URL associated with them so that when the user points such a location and clicks the left mouse button, the request to display the URL is sent to the web browser. Both tags’ string field has a format of “%d:%d:%d,%d:%d:%d %d:%d:%d,%d:%d:%d,%s,” 
where %d stands for an integer and %s for a string of characters composing a valid URL. The meaning of geographical coordinates coded in the first part of the string field varies. For GIS_RectArea, they correspond to NW and SE corners of the hot area rectangle. For GIS_CircleArea, they are the center of the circle and one of the points on the circle boundary.

- GIS_Object provides the means to incorporate external VRML/Open Inventor models into the GIS virtual world. GIS_Object tag is the only tag that is not an SoInfo node. It is supposed to be a descendant of SoSeparator, and must have exactly two or three children:

- SoInfo, which string field describes the geographic coordinates (longitude, latitude, altitude) of the object to be placed in the scene graph; string’s format should be “%d:%d:%d,%d:%d:%d %d,” where %d is an integer. After parsing this tag is replaced with an SoTranslation node.

- SoGroup, or its descendant, containing arbitrary set of children describing the object’s appearance. This may be any VRML/Open Inventor model of a plane, ground vehicle, building, etc.

- Optionally, another SoInfo node, with a string field containing a valid URL to be sent to the web browser if the user selects this object by clicking on it.

- GIS_Terrain represents a single terrain chunk, corresponding to one elevation samples file and one set of textures. This is a basic building block of the terrain model. Those blocks can be combined to create terrain chunks covering bigger areas. A set of parameters contained in the string field (of the format “%d:%d:%d,%d:%d:%d %d %f %d”) of this SoInfo node is the following:

- Geographic coordinates of the northwest corner of the terrain chunk

- Size of the terrain chunk in minutes. Currently our GIS server contains 15- and 30-minute files for most New York State.

- Mesh simplification threshold. A floating-point number passed to the MeshSimplifier object. The higher value the smaller number of triangles is created to produce an approximation of the terrain shape, but the faster rendering. Useful values vary between 0.3 for a very precise terrain shape approximation to 1.5 for rough approximations. The SceneBuilder object sets this value to 0.6.

- Preferred texture size in pixels. This can be used to force a wavelet decompressor to produce uncompressed textures smaller than their originals. For example, images of size 128 by 128 pixels may be created instead of 512 by 512 pixels in order to improve rendering speed, at a price of much worse visual quality of the rendered scene. This value is set to 512 by SceneBuilder object, which is most often the maximum texture size allowed by OpenGL libraries on SGI systems. If OpenGL receives requests to map textures bigger than 512 by 512 pixels, it anyway downsamples them to this size, what takes time and worsens the quality.

The location of texture files on the HTTP server is deduced by concatenation of: GIS_Root tag value, string “/Textures/”, string representing the integer value of chunk size in minutes, and string corresponding to the coordinates of northwest terrain chunk corner. The location of elevation samples is created similarly, by concatenation of GIS_Root, “/DEM/”, chunk size, and finally the coordinates of northwest terrain chunk corner. Appropriate extensions are appended to those strings: “dwv” or “dem” (wavelet compressed or not) for elevation samples files, and “twv” or “ppm” for textures (compressed on not).

To exemplify, we will analyze a simple scene description file. The code below may be a SDF available on some HTTP server, or it may represent in Open Inventor text interchange format a scene graph created by a SceneBuilder object:

#Inventor V2.0 ascii

Selection {

Environment {

fogType SMOKE

fogVisibility 5000.0

fogColor 0.6 0.6 1.0

}

ShapeHints {

creaseAngle 3.14

}

DEF GIS_Root Info {

string "http://sandman.npac.syr.edu:8888/GIS"

}

DEF GIS_Origin Info {

string "76:15:00,43:15:00 0"

}

DEF GIS_Bounds Info {

string "76:15:00,43:15:00 75:45:00,42:45:00"

}

DEF GIS_Scale Info {

string "12000 12000 3"

}

DEF GIS_Texture Info {

string "MAP,Map (Scale 1:250 000)"

}

DEF GIS_Texture Info {

string "SAT,Satellite Image (Resolution 30m)"

}

DEF GIS_Terrain Info {

string "76:15:00,43:15:00 15 0.5 512"

}

DEF GIS_Terrain Info {

string "76:0:00,43:15:00 15 0.5 512"

}

DEF GIS_Terrain Info {

string "76:0:00,43:0:00 15 0.5 512"

}

DEF GIS_Terrain Info {

string "76:15:00,43:0:00 15 0.5 512"

}

DEF GIS_RectArea Info {

string "76:15:00 43:10:00 76:00:00 43:00:00,http://www.sybercuse.com"

}

}

As we can see, GIS_Root value is http://sandman.npac.syr.edu:8888/GIS. Thus, all texture files and elevation data files will be located on this server. GIS_Origin and GIS_Bounds tags set the center of Open Inventor’s coordinate system to 7615’W 4315’N, and restrict the viewer’s camera position to the inside of the area delimited by 7615’W to 7545’W and 4315’N to 4245’N.

GIS_Scale tag fixes 1 degree of latitude and longitude to take 12000 Open Inventor units, and 1 meter of altitude to 1/3 unit.

One “hot spot” area has been defined: a rectangle roughly covering Syracuse, NY (from 7615’W to 7600’W and from 4310’N to 4300’N). The GIS viewer, if the user clicks on the terrain covered by this area, will launch a request to an HTML web browser to display the URL http://www.sybercuse.com.

There are four GIS_Terrain tags. GIS_Terrain string field of the first of them contains the text: “76:15:00,43:15:00 15 0.5 512.” Accordingly, 3D GIS viewer will try to download elevation samples from the following URL:

http://sandman.npac.syr.edu:8888/GIS/DEM/15/76:15:0,43:15:0.dwv.

In case of download failure, uncompressed version of the file will be downloaded:

http://sandman.npac.syr.edu:8888/GIS/DEM/15/76:15:0,43:15:0.dem.

Two GIS_Texture tags have been found, “MAP,Map (scale 1:250 000)” and “SAT,Satellite image (Resolution 30m),” therefore the texture files for this terrain chunk would be:

http://sandman.npac.syr.edu:8888/GIS/Textures/MAP/15/76:15:0,43:15:0.twv (or .ppm)

http://sandman.npac.syr.edu:8888/GIS/Textures/SAT/15/76:15:0,43:15:0.twv (or .ppm)

## 16 Web Resource Class and Its Use

Since our GIS viewer was designed as a specialized web viewer, one if its most important components is a mechanism for accessing resources over the Internet: WebNode. WebNode is used to download any arbitrary object described by a valid URL. In case of our system those objects include SDF, elevation data files, texture files, and hypertext documents created by GIS database.

Here is the declaration of the WebNode class:

class WebNode

{

public:

typedef void CallbackFunc( WebNode* node, void* usedData);

WebNode( const char* location, CallbackFunc cb = NULL,

void* ud = NULL);

~WebNode( void );

char* getBuffer( void ) { bufGiven_ = true; return dataBuf_; }

int getBufSize( void ) const { return dataSize_; }

operator const void* ( void ) {return (const void*) !intStatus_;}

//...

}

The simplest way of using WebNode object in an application is constructing the object and obtaining the downloaded resource from the WebNode’s buffer:

if (WebNode* wn = new WebNode(“http://sandman.npac.syr.edu:8888/GIS”))

{

//download successful

resource = wn->getBuffer();

//other actions

} else {

//download failure

cerr << “WebNode failed: “ << wn->getErrMsg() << endl;

//other actions

}

WebNode class has been designed in the way flexible enough to be used for both synchronous and asynchronous transfer of data through the Internet socket connection. If the constructor is provided with only one parameter (string containing the URL to download), the transfer will be synchronous, i.e. the control is given back to the thread after the resource has been received and the server closes the connection. This mode is deployed while downloading the SDF and elevation files.

However, if a pointer to user’s callback function has been passed to WebNode constructor, the download method does not wait until whole data has been received. Instead, it registers its method handle_input by XtAppAddInput X Toolkit intrinsic call, so that handle_input is called whenever data from the server arrives and therefore WebNode works in asynchronous mode. User’s callback function is called when the transfer is completed, so that the application is notified. Our system takes advantage of this feature when downloading textures to be displayed on top of terrain surface and sending the requests to the GIS database. In this way such requests do not block user’s interaction with the system.

## 17 Link to GIS Database

We implemented access to GIS database according to the Web concept: via requests expressed in the HTTP protocol. For retrieval of simple information, such as distance to the nearest airport or school, we send an HTTP request to the Illustra Webdriver module, and parse the answer ourselves in the main 3D viewer, an instance of SoXtWebViewer object. The request passes to Webdriver the geographical coordinates of the location, a type of feature we are interested in, and the maximum distance from the location we accept.

Webdriver to answer this request launches a SQL query defined by the following code, embedded in HTML page:

<?MISQL SQL="

select

Feature_Name,

Geocode,

distance(Latitude,Longitude,$select_lat,$select_lon)::integer

from Features

where Feature_Type like '$feature%'

and distance(Latitude,Longitude,$select_lat,$select_lon) 
        < ($radius)*100 ;">

{$*	}

<?/MISQL>

Every time the user requests information on a particular location, 3D GIS brings a new Netscape window with results of a database query and a HTML form, which lets query the GIS database in more complex fashion.

Sending URLs to a web browser has been implemented via remote control of Unix version of Netscape Navigator. When Netscape Navigator is invoked with the -remote argument, it does not open a window, but instead connects to and controls an already-existing process. The argument to the -remote switch is an Xt action to invoke, with optional arguments.

Remote control is implemented using X properties, so the two processes need not be running on the same machine, and need not share a file system.

## 18 Collaborative terrain viewing

From Paul’s thesis

# 19 Installation

The up-to-date version of 3D GIS can be downloaded from NPAC from the following URL: http://gis.npac.syr.edu:8888/GIS/3dGIS.tar.gz. Installation of the package involves two stages: placing the software in the location TANGO daemon can find it, and setting up the necessary environment variables. The first part is quite straightforward, since the location of the 3D GIS files is determined by the value of the TANGO_APP_ROOT environment variable. The following commands should install the software:

cd $TANGO_APP_ROOT

gunzip -c /tmp/3dGIS.tar.gz | tar xvf –

assuming of course that the downloaded archive was stored in /tmp directory. The other part of installation will be discussed in detailed fashion in Section 21 Environment.

## 20 System Requirements

3D GIS requires an SGI workstation capable of running Open Inventor software. These include older models, such as Indy, and all the models available from SGI currently. As far as operating system is concerned, 3D GIS requires SGI IRIX, release 5.3 or above. Open Inventor Execution Only Environment component has to be installed. Like any other 3D graphics software, for 3D GIS to render terrain smoothly, quite large amount of operating memory has to be available. Displaying areas of size 70 by 70 miles with 100 meters resolution of elevation grid and one 30 meters resolution texture is possible only with 64 MB free memory available exclusively to the 3D GIS process.

## 21 Environment

The installation package for 3D GIS comes with a simple C-shell script that takes care of setting up all the environment variables necessary for proper operation of the software. To accept default settings it is sufficient to add the following lines to the user’s ~/.cshrc file:

pushd $TANGO_APP_ROOT/GIS

source gisenv

popd

Customization is possible by adjusting some of the settings defined in gisenv file:

- GIS_VIEWER_PATH – this variable defines the path to the actual 3D GIS viewer, not the mapping window user gets when he/she starts 3D GIS. Changing value of this variable may be useful if there are some problems with the system and a debug version of the 3D viewer is necessary to record detailed event log, or when the 3D viewer has been installed in a non-standard place.

- GIS_SB_GIS_ROOT – the URL pointing to the top level of the GIS data server file hierarchy. This variable will be most likely modified when the whole data repository with terrain elevation, satellite images and maps is installed on-site with 3D GIS. Otherwise, terrain data for models generated by selecting areas on the main mapping window, will be downloaded the default location, namely http://GIS.npac.syr.edu:8888/GIS.

- Parameters related to the state/area map used in the main mapping window. These include: GIS_MAP_PATH pointing to the location of the XPM image file; GIS_MAP_CHSIZE defining size of a single map quad in geographical minutes; GIS_MAP_CHHPIX and GIS_MAP_CHVPIX determining how many pixels one quad takes on the image in horizontal and vertical direction, respectively; GIS_MAP_NWLON and GIS_MAP_NWLAT – which are the geographical coordinates of the upper left corner of the map.

- GIS_CACHE_DIR – with default value of “/tmp,” determines where 3D GIS should store its temporary cache files. It is important to make sure that there is large amount of free disk space available on the file system pointed by this variable (order of tens of MB).

- GIS_LOG_DIR – defines where 3D GIS will store its debug information (useful only if the debug version of the 3D viewer has been indicated by GIS_VIEWER_PATH).

It is very important to include the directory 3D GIS has been installed in somewhere in the value of LD_LIBRARY_PATH environment variable. Otherwise, the 3D viewer will fail to locate the necessary shared libraries, and effectively it will not be able to load successfully.

# 22 User Guide

## 23 Mapping Window

When the user starts a new session of 3D GIS by pressing a button labeled ‘GIS’ in the TANGO Interactive Control Application, the main 3D GIS mapping window pops up, displaying a map of the area available for viewing in 3D viewer windows. The map itself can be customized by changes to environment variables defined in gisenv file in the main 3D GIS directory (see Section 21 Environment).

Figure 16. Main mapping window of 3D GIS

The button labeled “Launch” is used to start a new instance of 3D viewer. It is enabled only if the user is the master of the 3D GIS TANGO session. If there are other participants in the session, the same window will appear on their screens.

When there are any 3D GIS viewers started from the mapping window, selecting an area by dragging a mouse over it on the map will send a message to the 3D viewers notifying about a new world to be loaded and constructed. Remote 3D viewers that have been started by 3D GIS on machines of other session participants are also among recipients of this message.

The button “Quit” is used to close the session of 3D GIS.

## 24 3D Viewer

The main 3D viewer window is equipped with all the standard buttons, present in most of Open Inventor applications. The buttons on the right side of the window there are (from top to the bottom):

- Picking mode switch,

- Viewing mode switch,

- Open new world,

- Go to home camera position,

- Set current camera position to be the home position,

- View as much of the model as possible,

- Directional zoom.

The viewer works in two main operation modes: viewing (also referred to as navigating) mode and picking (pointing or selecting) mode.

In viewing mode mouse operations lead to changes to the position of the scene camera. Dragging the mouse while keeping the left mouse button depressed changes camera’s X and Z coordinates (latitude and longitude). Dragging the mouse while the middle button is depressed will change the altitude of the camera. Changing camera’s orientation is possible by pressing both the left and the middle mouse buttons and then dragging the mouse.

In picking mode clicking on a component of the scene may result in different actions, depending on the type of the object being clicked on. Clicks on the terrain itself launches a set of queries to the GIS database regarding the location and name of the closest to the selected location school, hospital, military base, airport, lake, town, etc. Also, Netscape Navigator window will pop up, and display results of a broader database query about this point. From that window the user has a chance to retrieve additional data using an HTML form.

Figure 17. 3D viewer window

When a user clicks on an external object (plane, building, tank, etc.) the URL associated with this object is sent to Netscape.

“Open New World” button brings to the user a dialog window where any valid URL can be typed in. If the URL points to a correct 3D scene description, the new world will replace the currently displayed 3D scene.

The button marked with the letter “L” (like “layer”) on the left side of the 3D viewer window is used to choose one of the available textures. When a new world is loaded to the viewer it is displayed with no textures by default. Rationale behind this design decision was high cost of downloading and processing textures, which may not be the ones the user wants to see. It may happen that the user is interested in examining terrain with maps, and the default texture was set to satellite images, or vice versa. When the user clicks on the “L” button, a small window appears with a list of all available textures to be displayed. Double clicking on one of the list items selects the texture to be rendered.

# 25 References

[1]	United States Department of the Interior. U.S. Geological Survey. US GeoData Digital Elevation Models. Data Users Guide 5. Reston, Virginia, 1993.

[2]	Josie Wernecke. The Inventor Mentor. Programming Object-Oriented 3D Graphics with Open Inventor, Release 2, Addison-Wesley, 1994.

[3]	ER Mapper Applications. Earth Resource Mapping, San Diego, California, 1995

[4]	Dave Mount. Analysis of Algorithms. University of Maryland, College Park, Maryland, 1994.