CIS 637

Homework

Tomasz Stachowiak

8.10

(a)

paged memory reference time = 2 x 200 = 400 ns

(we need to access memory twice: first to check page location, second to access page itself)

(b)

effective memory reference time =  0.75 x 200 	+	 0.25 x 400 = 250 ns

(page table hit – one access)	(no hit – two accesses)

8.17

(a)

- Select LDT or GDT based on selector g bit

- Based on selector s bits find table entry

- Check if offset is within limit

- If yes, add base to offset creating linear address

- Based on directory bits of linear address find entry in first level page directory

- Based on page directory pointer find page table.

- Based on page bits of linear address find entry in page table.

- Based on pointer from page table find page frame.

- Based on offset bits of linear address  find entry in page frame – it includes physical address

(b)

Hardware advantages:

- moves the time consuming process of address translation from slow  software to fast hardware

- reduce complexity of operating system

- introduce effective way of memory management (pages)

- provides programmers with logical memory independence (it is mapped to physical memory automatically)

- provides convenient way of accessing memory through segments

- allows memory segments sharing (GDT)

- page tables can be swapped allowing more memory to be used

- segment registers increase conversion for frequently used addresses

- introduces protection mechanisms

- logical address space is much higher than physical

(c )

Disadvantages:

- memory accesses overhead

- internal fragmentation

- performance decrease connected with page tables swapping

- hardware complexity increase