NORTHEAST PARALLEL ARCHITECTURES CENTER AT SYRACUSE UNIVERSITY

VIDEO ON DEMAND

- report

Tomasz Stachowiak

Syracuse 1996

TABLE OF CONTENTS

1. Introduction

2. Technology

2.1 H.263 video compression

2.1.1 H.263 vs. H.261

2.1.2 Negotiable options

2.2 Audio compression

2.2.1 GSM 06.10

2.2.2 Intel/DVI ADPCM

3. Project

3.1 Audio-video client

3.1.1 VOD system architecture overview

3.1.2 Overview

3.1.3 New elements

3.1.4 Synchronization

3.1.5 Random access

3.2 H.263 stream offsetting

# Introduction

Video On Demand(VOD) project is just a part of my internship in NPAC. My primary responsibility is Conferencing and Collaboration System but because I needed to implement video and audio synchronization algorithm for videoconferencing tools and I met some problem with starting this work at the conference level I decided to do it first in some better known environment such as VOD system. Additionally I could create tools which would be then integrated with this project. After that I made also some changes and improvements in existing VOD software connected with video and audio compression algorithms such as H.263, GSM 06.10, ADPCM.

# Technology

## H.263 video compression

### H.263 vs. H.261

With H.263 it is possible to achieve the same quality as H.261 with 30-50% of the bit usage. Most of this is due to the half pel prediction and negotiable options in H.263. There is also less overhead and improved VLC tables in H.263.

### Negotiable options

- Unrestricted Motion Vector mode. In this mode motion vectors are allowed to point outside the picture. The edge pels are used as prediction for the "not existing" pels. With this mode a significant gain is achieved if there is movement along the edge of the pictures, especially for the smaller picture formats. Additionally, this mode includes an extension of the motion vector range so that larger motion vectors can be used. This is especially useful in case of camera movement.

- Advanced Prediction mode. This option means that overlapped block motion compensation is used for the P-frames. Four 8x8 vectors instead of one 16x16 vector are used for some of the macroblocks in the picture, and motion vectors are allowed to point outside the picture as in the UMV mode above. The encoder has to decide which type of vectors to use. Four vectors use more bits, but give better prediction. The use of this mode generally gives a considerable improvement, especially subjectively because OBMC results in less blocking artifacts.

- Syntax-based Arithmetic Coding mode. In this mode arithmetic coding is used instead of VLC coding. The SNR and reconstructed frames will be the same, but generally fewer bits will be produced. The average gain for inter frames is 3-4%. This gain depends on the sequence, the bitrate and other options used. For intra blocks and frames, the gain is higher, on average about 10%.

- PB-frames mode. A PB-frame consists of two pictures being coded as one unit. The name PB comes from the name of picture types in MPEG where there are P-pictures and B-pictures. Thus a PB-frame consists of one P-picture which is predicted from the last decoded P-picture and one B-picture which is predicted from both the last decoded P-picture and the P-picture currently being decoded. This last picture is called a B-picture, because parts of it may be bidirectionally predicted from the past and future P-pictures. For relatively simple sequences, the framerate can be doubled with this mode without increasing the bitrate much. 
For sequences with a lot of motion, PB-frames do not work as well as B-pictures in MPEG. This is because there are no separate bidirectional vectors in H.263, the forward vectors for the P-picture is scaled and added to a small delta-vector. The advantage over MPEG is much less overhead for the B-picture part, which is really useful for the low bitrates and relatively simple sequences most often generated by videophones.

These options are negotiable. This means the decoder signals the encoder which of the options it has the capability to decode. If the encoder has any of these options, it can then turn them on, and for each of the options used the quality of the decoded video-sequence will increase.

## Audio compression

### GSM 06.10

GSM is a telephony standard defined by the European Telecommunications Standards Institute (ETSI). The GSM 06.10 compressor models the human-speech system with two digital  filters and an initial excitation. The linear-predictive short-term filter, which is the first stage of compression and the last during decompression, assumes the role of the vocal and nasal tract. It is excited by the output of a long-term predictive (LTP) filter that turns its input--the residual pulse excitation (RPE)--into a mixture of glottal wave and voiceless noise. GSM encoder compress 160 16-bit voice samples into 264-bit gsm frame. GSM 06.10 is faster than code-book lookup algorithms such as CELP. It offers 13kbps bandwidth.

### Intel/DVI ADPCM

ADPCM compression algorithm uses the correlation between adjacent audio samples to reduce bit rate. It transmits only the differences between samples and their predicted values which have less dynamic range than the samples themselves. Predictor coefficients and reconstruction levels are calculated dynamically using coded signal. It allows to reduce the bandwidth but make this adaption technique more susceptible to transmission errors. There are a few ADPCM standards like e.g. Intel/DVI  or G.721. Intel/DVI is not very computationally intensive still having good quality even for the music.

# Project

## Audio-video client

### VOD system architecture overview

VOD system consists of three major parts:

- Stream server - provides the stream of data from one of the server’s files, allows to start and stop transmission at any time and also to random access the data

- Database - provides the information about the movie available. It includes server address, file path, file length, frame rate, frames offsets and others

- Clients - read data from the server. It is the client responsibility to process the data from server in proper way based on information from database

VOD system architecture is shown on figure1.

Communication between database and clients is performed via Nestcape Navigator using standard HTTP, CGI mechanisms. Data from the server are transmitted using socket connections (TCP). Transmission control is done by additional control connection based on VOD client-server protocol.

Figure 1 VOD System architecture

### Overview

AV client is implemented for the SGI Indy, IRIX 5.x platform. It uses H.263 video compression, and either GSM or ADPCM audio compression. It supports QCIF and CIF file formats. It allows to play the movie, stop it and random access the movie.

### New elements

In existing movie formats audio and video data were included in the same stream. But what we wanted to do was to synchronize AV data from two independent streams. Hence it was necessary to open two server connections, send and receive control messages from two separate channels. Unfortunately database system offers information just about video stream. Therefore audio configuration had to be included in the data file. It was done by adding special audio header whose structure is presented below in table1

Table 1 Audio header

| Name | Length (in bytes) | Description |
| --- | --- | --- |
| Title | 12 | String “npac-audio”. Indicates that file is NPAC audio stream |
| Rate | 2 | Rate in samples per second |
| Channels | 1 | Channels number: 1 - for mono 2 - for stereo |
| Sample width | 2 | Sample with in bits |
| Code format | 10 | String indicating compression type “adpcm”, “gsm” supported |

### Synchronization

Synchronization is based on the internal SGI Audio Library mechanisms. Procedure sending audio samples to the speaker port blocks until all previous sent samples are played. In this case it is enough to send audio portion and then decode video frame to achieve the synchronization and keep the frame rate. The only problem that this requires single video frame decoding which moves the responsibility of single frame reading from the decoder to the AV client.

To obtain this effect  some changes were necessary both at the decoder and client side:

- the original Telenor’s H.263 decoder was transformed into library that allows decoder initialize and then decode single frame passed as function argument

- special function detecting H.263 frame end was implemented and use in single frame reading procedure

- to make processes of reading data from socket and taking the single H.263 frame independent a system of ring buffers was implemented

### Random access

Since audio portions take always the same size it isn’t a problem to find beginning of the portion knowing its size. Situation differs in H.263 video compression. First, H.263 frames have different length, second because it is predictive compression only INTRA frames can be accessed in this way. The way of solving this problem is described in chapter H.263 stream offsetting.

## H.263 stream offsetting

H.263 system is supposed to be used as a preview tool to the MPEG movies. Hence H.263 sequence will be acquired by converting MPEG files. As a preview tool it is absolutely necessary to have the capabilities of random access. It implies necessity of offset files generation.

To obtain offset files it was necessary to add some new features to the H.263 encoder:

- possibility of encoding INTRA frame every fixed number of frames (only INTRA frames can be accessed randomly)

- writing offset file with number and stream position of every INTRA frame

This mechanism allow to generate offset file parallel with movie converting