Development Of Integrated, Web-based Videoconferencing System

Northeast Parallel Architectures Center

Internship Report

Tomasz Stachowiak

Syracuse 1997

Table of contents

Background

Overview

Video

Video formats

Video compression

Introduction

Audio

Digital audio

Audio compression

ADPCM

GSM 06.10

Desktop videoconferencing systems

ITU-T H.320

Picture Resolution

Frame Rate

Pre and Post Processing

Motion compensation

Audio consideration

Data rates

Existing Internet conference products

Overview

Conference management

Conference participants

Conference applications

Architecture

Conference Engine

NCS API

Session API

Application API

Conference Manager

NCS Server

Applications

Video Tool

codec improvement

Frame rate control

Video format conversions

Audio Tool

Multiplexing

Buffering

Whiteboard

Overview

Implementation

NCS-Tango integration

Future work

Overview

Architecture

Audio-Video H.263 Client

New elements

Synchronization

Random access

stream offsetting

Future work

Introduction

Chapter

1

# Background

This report has been created during internship in Northeast Parallel Architecture Center at Syracuse University as a partial fulfillment to Master Degree program of Franco-Polish School Of New Information And Telecommunication Technologies (EFP) in Poznan, Poland.

Internship took place between July and December 1996 under supervision of Dr Marek Podgorny, NPAC Associate Director and was a part of NPAC-EFP internship program created by Marek Podgorny and EFP professor Czeslaw Jedrzejek.

# Overview

Report presents the results of the internship which consists of two projects:

- NPAC Conferencing System - design and implementation of multiplatformed, web-based, desktop videoconferencing system including variety of collaborative applications (e.g. Video Tool, Audio Tool, Whiteboard)

- Video On Demand - implementing of Audio-Video client for existing NPAC VOD architecture, using latest video and audio technology(e.g. H.263 video codec)

First part of this report describes briefly technologies used in development of both projects including existing software products performing similar tasks. Second part is dedicated to work done during project development stage and existing NPAC development environment.

Since both projects consist of state-of-the-art technologies, often emerged during development and most of them are in the research stage – it was impossible to create clear internship goals at the beginning. It results in some changes in the internship scope,  especially concerning technologies used.

Technology

Chapter

2

# Video

## Video formats

To represent any color distinguished by a human eye three primary components are necessary. This phenomena is widely used in television and video technology. Mixing lights of three color components: red, green and blue(RGB) it’s possible to display colors. However more efficient way of representing video signal is isolating luminance signal responsible for brightness and two chrominance components describing colors. This technique is particularly useful because human eye is less sensitive for colors than for brightness. Thus it is possible to reduce amount of information in chrominance components without affecting the image quality. Another advantage of this format is concentration most of the image information in luminance that results in reducing elements correlation. Hence components can be compressed separately without much loss in efficiency.

Television standards use different type of color spaces. In European’s PAL and SECAM  there is YUV color space, in NTSC there is YIQ. In both the Y component (luminance) is the same. For digital applications the equivalent of YUV is YCbCr where Cb and Cr correspond to U and V components respectively. It allows normalization of color components ranges. If luminance range is between <0; 1> the UV range is undefined but Cb and Cr ranges are between <-0.5; 0.5>.

The conversion between RGB and YCbCr formats can be represent as following matrices:

YCbCr=0.2990.5870.114−0.1687−0.33130.50.5−0.4187−0.0813RGB

RGB=101.4021−0.34414−0.7141411.7720YCbCr

Additionally H.261, H.263 and MPEG conversions perform slightly different scaling. According to CCIR Recommendation 601 color space is rescaled and shifted in following way: Y’ = 219/255 * Y + 16, Cb’  = 224/255 * Cb + 128, Cr’ = 224/255 * Cr + 128.

## Video compression

### Introduction

Uncompressed digital image is usually to large to effectively store, treat or transmit through the network. Therefore many compression techniques are applied to video images to decrease its size. However requirement for efficient video codec are very high - it must have high compression ratio,  keep the image quality and be fast. Additionally, lossless  compression methods such as Huffman, Arithmetic Coding or LZW do not work well on the images. Therefore it was necessary to create compression techniques designed specially for video. Recently the most popular algorithms are based on the ITU H.261 recommendation.

H.261 compression uses two different techniques enabling effective video compression:

- spatial redundancy removal – both INTRA and INTER frames coding

- temporal redundancy removal – INTER frames coding

#### Spatial redundancy removal

- Figure 1 INTRA frame coding

Spatial redundancy removal is method based on the Discrete Cosine Transform(DCT). YUV image is divided into blocks 8x8 pixels each. On each block DCT is performed according to following equations:

F[u,v]=14i,jL(u)L(v)cos(2i+1)up16cos(2j+1)vp16f(i,j)

L(x)=12forx=01otherwise

After DCT consequent block values represent information of increasing signal frequency regarding proper axis. Such transformation could be also obtain using FFT(Fast Fourier Transform) but FFT is less error robust what is big disadvantage for network transmission. Since human eye is less sensitive on higher frequencies it is possible to eliminate them without losing the quality.

Such elimination or actually reducing the component information is obtained during quantization. Each block value is divided by the proper constant collected in quantization table. There are different quantization tables for luminance and chrominance signals. Scaling quantization table by certain factor increase or decrease image quality. Additionally, custom quantization tables can be used and put into the image header.

The next step of compression is Zig-Zag scan. It converts two dimensional 8x8 block into 1x64 vector, grouping low frequencies in top of the vector. On such prepared vector standard RLE(Run Length Encode) and Huffman compression algorithms are performed.

- Figure 2 Zig-Zag Scan

#### Temporal redundancy removal

Temporal redundancy removal technique uses the correlation between two consecutive frames in video stream to reduce its size. To obtain this goal encoded H.261 stream consists of two types of video frames:

- INTRA (I-frames) -- coded only with spatial redundancy removal technique

- INTER (P-frames) – “predicted”, coded based on the “pseudo-difference” from previous frame

The first stage of INTER frame coding is motion vector estimation. For each macroblock  in coding frame (macroblock is structure consisting of four luminance blocks and two chrominance blocks – Cr and Cb) the area of the previous frame(reference frame) that match it best is found. Then instead of coding the macroblock – it is enough to code the difference between the best match area and add the information about motion vector to the compressed stream. Such information can be put into the macroblock header. The difference is then codec using spatial redundancy removal, the only difference is separate quantization tables for INTER frames.

#### Bitstream structure

As it was mentioned above to efficiently compress video stream -- apart from video signal data we need also some additional information. Thus,  it was necessary to create H.261 video stream structure to enable retrieving those information by the decoder. Simplified scheme of such structure with brief description of its fields is presented below.

- Figure 3 H.261 Bitstream structure

| Field | Description |
| --- | --- |
| PSC | Picture Start Code, unique sequence to delineate boundaries between pictures |
| TR | Temporal Reference, timestamp used for audio synchronization |
| Ptype | Picture type (INTRA / INTER) |
| GOB | Group of Blocks |
| Grp # | Group number, enables skipping the whole group indicating its number |
| Gquant | Group Quantization Value - common value used for quantization of whole group |
| Addr | Address of macroblock in case it is exact match |
| Type | If good match cannot be found block may be coded in INTRA mode |
| Quant | Quantization Value for macroblock |
| CBP | Coded Block Pattern, bitmask indicating which blocks are present in case they matched poorly |

#### Motion vector search

Motion vector estimation is the most computational intensive part of H.261 compression. Hence it is very important to implement effective method of search. So far multiple algorithms were used for this purpose. Usually faster method means lower video quality.  Two most important motion estimation  algorithms are:

- full spiral search -- checks all the positions in searching area starting from the middle and moving spirally outward

- two-dimensional logarithmic search – similar to binary search, checks only a few locations in the area then continues searching around the location that matched best, logarithmic search is the least computational  intensive, however it does not provide highest quality

H.263 is improved version of H.261 compression algorithm. The idea of image coding is the same but some additional element were added which significantly increase the compression efficiency. Achieving the same quality it was possible to reduce bit stream size of 30-50%. The main factors that enabled this significant progress are:

- half pixel motion vector prediction

- negotiable options

- improved (VLC) tables

#### Negotiable options

These options can be switched on by encoder if decoder signals that it is capable to decode them. Encoders should include all of them because each one can significantly increase the effectiveness.

- Unrestricted Motion Vector (UMV) mode. In this mode motion vectors are allowed to point outside the picture. The edge pixels are used as prediction for the "not existing" pixels. With this mode a significant gain is achieved if there is movement along the edge of the pictures, especially for the smaller picture formats. Additionally, this mode includes an extension of the motion vector range so that larger motion vectors can be used. This is especially useful in case of camera movement.

- Advanced Prediction mode. This option means that overlapped block motion compensation is used for the P-frames. Four 8x8 vectors instead of one 16x16 vector are used for some of the macroblocks in the picture, and motion vectors are allowed to point outside the picture as in the UMV mode above. The encoder has to decide which type of vectors to use. Four vectors use more bits, but give better prediction. The use of this mode generally gives a considerable improvement, especially subjectively because OBMC results in less blocking artifacts.

- Syntax-based Arithmetic Coding mode. In this mode arithmetic coding is used instead of VLC coding. The SNR and reconstructed frames will be the same, but generally fewer bits will be produced. The average gain for inter frames is 3-4%. This gain depends on the sequence, the bitrate and other options used. For intra blocks and frames, the gain is higher, on average about 10%.

- PB-frames mode. A PB-frame consists of two pictures being coded as one unit. The name PB comes from the name of picture types in MPEG where there are P-pictures and B-pictures. Thus a PB-frame consists of one P-picture which is predicted from the last decoded P-picture and one B-picture which is predicted from both the last decoded P-picture and the P-picture currently being decoded. This last picture is called a B-picture, because parts of it may be bidirectionally predicted from the past and future P-pictures. For relatively simple sequences, the framerate can be doubled with this mode without increasing the bitrate much. For sequences with a lot of motion, PB-frames do not work as well as B-pictures in MPEG. This is because there are no separate bidirectional vectors in H.263, the forward vectors for the P-picture is scaled and added to a small delta-vector. The advantage over MPEG is much less overhead for the B-picture part, which is really useful for the low bitrates and relatively simple sequences most often generated by videophones.

# Audio

## Digital audio

Frequencies perceived by human ear as sounds are typically between 20 Hz and 20 kHz. Human voice can produce frequencies between 40 Hz and 4 kHz. Sounds to be digitalized must be converted from the continuous time and values domain into the discrete one.

Process of converting it in time domain is called sampling. According to Nyquist’s theory signal has to be sampled with rate at least twice higher than its frequency. Typically computer audio cards offer sampling rates from 8 kHz to 44.1 kHz depending on required quality. 8 kHz sampling rate is enough for the human voice sampling.

Then audio signal is quantized - converted to discrete values. During this process some information are lost what results in appearing of quantization noise. Its influence on audio quality depends on the number of quantization levels. Increasing of quantiztion levels leads to necessity of coding audio sample on more bits. Usually this number vary from 8 bits (256 levels - voice quality) to 16 bits (65536 levels - CD quality).  Because human ear is not equally sensitive to signal changes at different level some quantization methods are not uniform(PCM), but logarithmic spaced. It improves audio quality without changing the sample size.

## Audio compression

Transporting uncompressed digital audio takes a lot of bandwidth. Therefore in most videoconferencing system various compression algorithm are used. Some of them apply to general audio signals, others are specifically designed for compressing speech.

### ADPCM

ADPCM compression algorithm uses the correlation between adjacent audio samples to reduce bit rate. It transmits only the differences between samples and their predicted values which have less dynamic range than the samples themselves. Predictor coefficients and reconstruction levels are calculated dynamically using coded signal. It allows to reduce the bandwidth but make this adaption technique more susceptible to transmission errors. There are a few ADPCM standards like e.g. Intel/DVI  or G.721. Intel/DVI is not very computationally intensive still having good quality even for the music.

### GSM 06.10

GSM is a telephony standard defined by the European Telecommunications Standards Institute (ETSI). The GSM 06.10 compressor models the human-speech system with two digital  filters and an initial excitation. The linear-predictive short-term filter, which is the first stage of compression and the last during decompression, assumes the role of the vocal and nasal tract. It is excited by the output of a long-term predictive (LTP) filter that turns its input - the residual pulse excitation (RPE) - into a mixture of glottal wave and voiceless noise. GSM encoder compress 160 16-bit voice samples into 264-bit GSM frame. GSM 06.10 is faster than code-book lookup algorithms such as CELP. It offers 13kbps bandwidth.

# Desktop videoconferencing systems

Desktop videoconferencing is interesting and much less expensive alternative for “room” based videoconferencing systems. In desktop videoconferencing members use just their computers similarly as telephones. From business point of view it has many advantages: reduce high travel costs, speed up communication, increase productivity. However nature of video data requires high performance equipment and high bandwidth networks what is certainly connected with high expenses. Unfortunately there are also other drawbacks: current systems interoperability problems, limited availability of transmission lines.

To provide potential users set of requirements which desktop videoconferencing system meets a few videoconferencing standards were created. The most important is ITU-T H.320 standard to which most videoconferencing systems are compliant. This chapter briefly describes videoconferencing standard H.320 and gives quick review of existing conference systems for the Internet.

## ITU-T H.320

H.320 is the ITU-T standard for videoconferencing. As the ITU-T develops standards, one goal is to define the minimum requirements all videoconferencing vendors must support. This minimum requirement ensures that all H.320-complaint systems will communicate with each other. The optional requirements affect the quality of the audio and video and their implementation is left to the manufacturer.

A videoconferencing system can belong to one of the following classes:

- Class1 - minimum level of support (required by H.320)

- Class 2 - support of some optional features

- Class 3 - support of all optional features

### Picture Resolution

In H.320 system there are two types of picture resolutions: Quarter Common Intermediate Format (QCIF) and Common Intermediate Format (CIF). Class 1 systems will only support QCIF, Class 2 systems may support CIF, and Class 3 will support CIF. The ability to support CIF resolution is critical to the quality of the picture. CIF capable systems provide a picture resolution of 352 x 288. QCIF provides quarter the resolution -- 176 x 144. A QCIF picture will look like an unfocused blocky image. A high-quality system will be forced to degrade its resolution if it connects to a low-quality system. For example, if a Class 1 and Class 3 systems connect in a call, the Class 3 system will be forced to display a QCIF resolution.

### Frame Rate

Frame Rate, also referred to as the Number of Frames Per Second (fps), is the number of times the picture image is refreshed. H.320 systems can support frame rates of 7.5, 10, 15, or 30 fps. A low frame rate will cause the motion to appear jerky. The higher the frame rate, the smoother the motion will be. Class 1 systems are required to support a frame rate of 7.5. Support for the higher frame rates are left to the manufacturers and the ones with a desire to deliver high quality products will support higher frame rates. Typically Class 2 systems will support up to 15 fps. Class 3 will support up to 30 fps. Unfortunately even the manufacturers who deliver Class 3 systems are affected by manufacturers who are not driven to produce high quality products. This means if a Class 1 system connects to a Class 3, the Class 3 system will have to downgrade its capabilities to communicate to the Class 1system. The Class 3 system would be forced to use 7.5 fps for the call to work.

### Pre and Post Processing

Pre-processing is a complex process to reduce the amount of re-coding the background. This is not required for Class 1 systems. If pre-processing is not used the video encoder can spend a lot of time encoding 'noise' caused by the poor lighting of the camera. These situations may make the system think that there is motion in the background when in fact there is none. Pre-processing ensures that only real motion gets encoded. Post-processing will compensate for picture degradation's caused by fast motion. Post-processing can reduce the blocking and noisy effects caused by H.320 video coders. Post-processing can also be used to enhance the frame rate and reduce the jerky motion effect caused by low frame rates.

### Motion compensation

There are two aspects of motion compensation: motion estimation and the actual motion compensation. Motion estimation is performed at the encoder to determine what the motion vector should be. Motion compensation is performed at both the encoder and the decoder, and consists of moving blocks of data around based on the motion vector. Essentially, motion compensation only encodes the section of video where motion has occurred rather then encoding the entire video area for every frame. This is especially important at lower bit rates. All H.320 systems must have the ability to decode a motion compensation signal. Providing encoded motion compensation is optional. The encode ability is where video quality improvements are made. A Class 1 system will not support the encode compensation, a Class 2 system may support a limited form of motion compensation, and a Class 3 system will support a higher form of encode motion compensation.

### Audio consideration

H.320 specifies three types of audio:

- G.711 -- 48-64 Kbps Narrowband

- G.722 -- 48-64 Kbps Wideband

- G.728 -- 16 Kbps Narrowband

A Class 1 system only has to support G.711. G.711 is telephone quality audio (narrow-band, 3khz). G.722 produces stereo-quality audio (wideband, 7Khz). A higher data rate, typically 256 Kbps and above, Class 3 systems will support G.722 offering the user the best audio quality available. Class 1 systems will not support G.722, and Class 2 typically will support G.722. G.728 (16 Kbps) is narrow-band audio but it is important for low bit-rate calls, typically less then 256 Kbps. For example, in a 128 Kbps videoconference (typically via BRI) a high-quality manufacturer will offer the user to select between G.722, 64 Kbps, or G.728, 16 Kbps. If audio quality is more important then video quality (64 Kbps for the audio will only leave 64 Kbps for the video), the user can choose to use G.722. If video quality is more important the user will use G.728. The audio will use 16 Kbps leaving 112 Kbps for the video. High-quality manufacturers will allow the user this choice, not dictate it to them. Class 3 systems again are limited by the lesser Class systems. If a Class 3 system establishes a call with a Class 1 system the audio will be G.711. The Class 3 system is forced to use lower-quality audio in order for the call to work.

### Data rates

Data rates have an impact on the perceived quality. The higher the data rate the better the video quality will be. At T1 speeds (1536 Kbps) the video quality will be optimal. Realistically most users will not run at this speed, this is mainly a cost issue. In today's market many users are using 768 Kbps as a trade-off between high quality video and cost. 384 Kbps is probably the most common data rate in use today, 128 Kbps is becoming more popular with the availability of ISDN. As desktop videoconferencing becomes more mainstream 128 Kbps will become one of the more common data rates.

Many factors affect the overall quality of an H.320 system. The ability of vendors to implement optional features will greatly impact the overall quality. The user should understand their environments and the application the H.320 system will be used in. These factors can help make a decision on what the requirements for the video conferencing system will be. Applications can range from desktops to boardrooms and each will have its own requirement. Not only will the system the user purchases determine the quality but the capabilities of the system on the other end will have a major impact to the overall quality. A lesser capable system will always force the high quality system to downgrade its quality in order for the conference to be held. The best solution for a user is to require a system that is flexible and software upgradable. Flexibility means the system can be configured to the requirements of the user, which may change over time or even may change from conference to conference. Software upgradability provides investment protection to the user by allowing the user to take advantage of new features without being required to buy new hardware.

- Table 1 H.320 conformance test

| Item | Class 1 | Class 2 | Class 3 |
| --- | --- | --- | --- |
| Picture Resolution | QCIF mandatory | CIF optional | CIF mandatory |
| Frame Rate | 7.5 fps | up to 15 fps | up to 30 fps |
| Motion Compensation | must be able to decode motion compensation signal | limited form of encode motion compensation | higher form of encode motion compensation |
| Audio | G.711 | G.722 | G.722, G.728 |

## Existing Internet conference products

The teleconferencing systems for the World Wide Web environment differ significantly from the commercial ones. The major reason is that here one can not assume a large network bandwidth available. This in turn impacts a video coding scheme, a picture size and a number of transmitted frames per second. Usually these systems are fully implemented in software and use more sophisticated video coding schemes in order to cope with insufficient network bandwidth available. There also has been a lot of work done in order to develop appropriate transport protocols (see section 5.4) which would be able to transfer the increasingly demanding amount of data in videoconferencing. As a result, an entire suite of protocols and applications have been developed which seek to alleviate the many problems that exists in video conferencing.

In the World Wide Web environment these are a multicast transport scheme and the real-time transport protocols (such as RTP, described in section 5.4.1) that enable a real-time multipoint multimedia conferencing. Along with the development of the MBone and multicast protocols (see section 5.4.3) it became feasible to run conferencing applications on the Internet. All conferencing tools for WWW use those technologies without which they would simply not be operational.

In the following paragraphs we describe the most popular teleconferencing packages currently in use.

Network Video (NV), developed by Xerox Park was one of the first network video only conferencing packages. The entire system is implemented in software and makes use of RTP and IP multicasting. This package uses a proprietary video coding scheme which gives a compression factor of about 20-35. The encoding of a one frame takes about tens of milliseconds and typical framerate equals to 10-15 frames per second (reasonable for a local area network with a typical workstation). The Haar wavelet transform and discrete cosine transform are used to encode those 8x8 pixels blocks which have changed significantly as compared to the previous frame. No motion estimation is employed. A user can increase a framerate by lowering the quality of coded pictures. This is mainly imposed by the network's inability to handle a large amount of video data. NV is capable of video capturing from an X11 window and therefore making itself compatible with many platform-dependent video grabbers. For more details on NV, please refer to [19] or [20].

Inria Videoconferencing System (IVS), developed by INRIA researchers was one of the first video and audio conferencing systems. The audio coding scheme in IVS gives a quite good quality and generated bitstream takes up to 64 kbps. Video data uses a mere 128 kbps. The images are usually a quarter screen or smaller. IVS uses H.261 video compression algorithm which due to its advanced motion estimation and compensation techniques reduces significantly the amount of data to be transmitted. The drawback is that due to its complexity H.261 encoding and decoding takes more time. Therefore, on a lower end its performance in video encoding is rather not sufficient. On the other hand,  IVS system is the only one available supporting synchronized audio and video in one application. The popularity of IVS package may grow with the appearance of the faster workstations. For more details on IVS, please refer to [19] or [20].

Videoconferencing Tool (Vic), developed by Lawrence Berkeley Laboratories, is perhaps the most flexible conferencing system available. It combines all that was gained by its predecessors, NV and IVS along with adding some new features of its own. It supports many more compression algorithms and has a user interface that make it particular useful. It is a video only tool and has to be used in conjunction with an audioconferencing tool  VAT(Visual Audio Tool) and session management tool SD(Session Directory). Vic supports the following video compression schemes: MPEG, JPEG, NV, CU-SeeMe, CellB and its proprietary intraH.261. Therefore, using the Vic tool, one can join many different conferences as long as they use one of the aforementioned compression algorithms. The innovation here is that instead of encoding the block changes in a picture, which depends on previous frames, this version of H.261 simply encodes the entire changed block. Vic takes advantage of the RTP protocol to coordinate many different compression schemes (RTP header can have any application specific extensions). A new feature called conference bus  is implemented in VIC. This allows all participants to receive critical information concerning a course of a teleconference (floor assignments, different conference formats). Another big advantage of VIC is that it is able to support different hardware codecs. Vic reaches a sending framerate of 30 fps at a bandwidth of 1.1 Mbps. Unlike NV, VIC is not able to capture video data directly from an X11 window and therefore is dependent on a video grabber type in use (Vic supports a very limited set of video grabbers). For more details on Vic, please refer to [19], [20] or [8].

NPAC Conferencing System

Chapter

3

# Overview

The goals of this project are:

- create infrastructure for collaborative, distributed, real-time applications (video, audio)

- implement multiplatform, modular conferencing system for Internet, compliant to existing standards (T.120, H.320)

- create development environment for improving and adding collaborative components

Initially such system was developed in NPAC based on InSoft's OpenDVE software development kit. Since there have been restrictions and limitations for OpenDVE-based conferencing system, we decided to develop our own conferencing software infrastructure. Reasons for creation NPAC Conferencing System (NCS), independent of OpenDVE:

- full control over data transmission process – especially important for real-time applications(video, audio)

- ability to porting the system on different platforms

- ability to create system compliant to conference standards(T.120, H.320)

- possibility of implementing standard real-time data transportation protocols(RTP, RSVP)

- ability to integrate NCS seamlessly with other NPAC projects(e.g. Tango)

- integrate the multimedia conferencing tools into the World Wide Web environment

First, NCS was implemented on SGI Indy workstation. However, after satisfied performance under IRIX 5.X operation system, NCS was ported successfully on PC Windows NT/95 platform.

# Conference management

A conference refers to a group of geographically dispersed nodes that are joined together and that are capable of exchanging audiographic and audiovisual information across various communication networks. Conference participants may have access to various  types of media handling capabilities such as audio only (telephony), audio and data, audio and video, or audio, video and data.

The conference in NCS is based on the notion of session. Session is a group of users on different machines connected in virtual many-to-many channel. So far there can only be one session on one machine an just one user on the machine can participate in conference. Many applications can be attached to the session.

## Conference participants

Conference participant can start new session and invite others. Invitation will be visible for remote user providing he has his Conference Manager running. He can also enable the Conference Engine “watchdog” function which automatically starts Conference Manager and displays incoming invitation. Remote user can accept or reject the invitation. If he accepts invitation he will become conference participant. From this moment all his NCS application will collaborate within the current session. Each conference participant is free to invite users to participate in conference. So far here is no other way to join the conference.

Each participant has his own user identification number, unique within the session. Every conference message includes this user id. There is always one special conference participant – statekeeper. Although all participants hold all the information about the conference it is necessary to name one that provides information to the new participants and solve any state conflicts. Usually it is conference initiator but in some situations (e.g. initiator left) it may be another conference member.

## Conference applications

Conference application is every application which is attached to Conference Engine and collaborates within the session. Each participant can start conference application. Applications can be started from Conference Manager control panel or independently. If application is invoked from Conference Manager panel it will be started automatically on all participant machines. Also exiting the application from Conference Manager panel causes closing all participant applications. If application is started independently Conference Engine may reject the application trial to connect to the session.

# Architecture

The main assumption during NCS development was its modularity. Modular system structure enables easy and quick modification and improvement of some of its parts without affecting the others. Based on core modules it is possible to add new collaborative applications using existing NCS mechanisms. It is especially important in Internet environment where system context evolves dynamically.

NPAC Conferencing System consists of two main modules. First module is the Conference Engine (CE) which contacts the remote hosts, processes control messages from them and passes the indications of events to the Conference Manager (CM). Conference Engine also processes the local messages from Conference Manager. To create Conference Manager the set of API functions is provided. There is also the API for the conference application management.

NCS uses UDP (User Datagram Protocol)  to speed up data exchange. For real-time applications it fulfills the performance requirements without affecting quality of service. It's up to application to create higher level, more reliable protocol if necessary. However implementing such a protocol for priority messages is also predicted as internal part of Conference Engine.

- Figure 4 NCS Architecture

## Conference Engine

Conference Engine is the core of NCS system. It performs the following functions:

- keeps the information about conference members and applications

- receives the requests and sends the indications to Conference Manager and conference applications

- exchange the control messages with other conference members

CE sends and receives all the conference control messages, however application data exchange is performed separately by the applications. It allows to avoid bottleneck  in CE which would be intermediate point for all NCS messages in opposite case. It is particularly important for real-time applications for which this system was designed. Therefore applications keep copies of some necessary conference information (e.g. members addresses) and they are indicated of any changes in that data by the CE. It is particularly important for real-time applications for which this system was designed.

Conference Engines on different hosts communicate with the UDP socket connections. Also communication with CM and local applications is done by the same type of connection and even by the same socket. To accomplish this special kind of control protocol is designed consisting information about message sender, receiver and type.

## NCS API

NCS API was created to facilitate development of NCS-based applications. Conference Engine and API provides the backbone for conferencing system. It allows flexible improvement of NCS, creation of variety new applications and enriching its functionality. According to type of application that is implemented there are two  different kinds of API functions:

- session API - for development of conference management applications (Conference Manager)

- application API - for development of ordinary conference applications (Video Tool, Whiteboard)

API functions perform all the message exchanging and offer easy to use callbacks mechanism to indicate of NCS events. They also enable acquiring all kinds of conference information that may be useful for application.

### Session API

First conference managing application  must initialize conference with ncsSessionInitialize()  function which returns error value upon initialization failure. Initialization open the control socket and informs the Conference Engine that managing application is active.

Then conference callbacks must be set with ncsSessionAddCallback() function. Callbacks are simple and convenient way of informing applications of  conference events. Developer write the procedures handling events and add them as callbacks to conference system.

To properly react on system events application developer must implement the checking loop. Depending on developing platform it may be XWindows working procedure, Windows thread or ordinary loop. Inside the loop he can obtained the application file descriptors set which may be used as a parameter for the standard socket select() command. If at least one of the file descriptors is active function processing incoming messages (ncsProcessIncoming()) should be invoked.

After that application may wait for invitation(which invoke callback function) or start new session (ncsSessionCreate()) and invite participants(ncsSessionAddMember()). NCS session API provide also several other procedures dealing with invitation handling, applications managing, obtaining necessary information.

### Application API

To collaborate within the current session application must attach to Conference Engine with ncsAppAttach() command which returns the application handle used in all other conference operations. Application API has similar callback mechanism to session API. It also handles the incoming messages in similar way. Additionally inside the loop function ncsAppGetData() must be invoked to retrieve incoming messages from other participants.

Application can obtain some conference information(e.g. its id, number of participants), send message to all conference participants or single one. Finally application should detach from Conference Engine (ncsAppDettach())

## Conference Manager

Conference Manager creates NCS user interface and interacts with user to manage the conference. It uses API session functions to:

- create and destroy session,

- inviting conference members,

- accepting or rejecting conference invitations.

Additionally it offers possibilities to starting and closing NCS application. Information about system application are included in easily configurable text file what ensure the required system flexibility.

## NCS Server

Server is optional part of NCS – system works properly without it. However, working with server is much more convenient, because server keeps track of all users with NCS started. This important information allows to avoid sending invitation to non-existing(from the NCS point of view) users. In the future functionality of the server will be extended of capabilities of giving other useful information about NCS users

# Applications

## Video Tool

Video Tool is an application implemented using NCS API over the backbone of Conference Engine and Conference Manager. Video Tool has the capabilities of:

- capturing, compressing and sending video stream to all conference participants

- receiving, decompressing and displaying video from other participants

Since it is the application designed for the Internet it adopts two very efficient video compressing techniques:

- H.261 – Standford University codec, improved and adapted for the needs of videoconferencing

- H.263 – Telenor’s codec, improved and adapted for the needs of videoconferencing by Janusz Bulawa

Both options provide very good picture quality. H.263 option takes less bandwidth but is more CPU intensive, thus for slower machines H.261 algorithm is recommended. Additional feature enables only INTRA frames compression which speed up coding process. However it causes significant increase of used bandwidth.

Video Tool automatically recognize received video format and uses appropriate decompression method. All the video frames are captured and sent in QCIF format, however it is possible to switch on the expand mode where QCIF frames are extrapolated to CIF format. Application includes statistics module which provides information about bitrate, framerate and other connection parameters.

### codec improvement

Initially Standford University H.261 encoder offered very poor performance. It was absolutely necessary to improve it for utilization with NCS. In motion compensation compressing algorithms the most computational intensive part is motion vector search. After extensive performance testing it turned out that motion estimation function takes 70% of compressing time. The algorithm used in Standford codec was full spiral search which is very simple but also inefficient. Thus we chose to change the motion estimation algorithm for faster one. The fastest motion search algorithm is 2D logarithmic search, however it offers worse quality. Since high quality in rather static conference images is not very important and performance is the most crucial factor we decided to use this method in our implementation. The source code for logarithmic search algorithm was taken from Berkeley MPEG encoder and adopted for our compressor. This improvement increased encoder performance almost three times.

Additionally we enable using H.261 encoder in “INTRA frames only” mode. It required removing the motion estimation part and decoding reference frame part. It speed up compression process of another 20%.

Apart from performance improvement H.261 codec required other changes to adapt it for videoconferencing purpose:

- converting executable program to a library

- retrieving the initialization procedures and encoding/decoding single frame functions

- converting i/o functions – so far they communicated only with the files, they were converted to pass the data in argument and return data as a memory pointer

- multiplying all the reference frame information for multistream decoding (streams from different participants)

- adding INTRA frame every fixed number of coded frames – necessary to prevent application crash if one frame is lost

### Frame rate control

Since encoding video stream is very CPU intensive, it was required to implement the frame rate control mechanism to allow CPU to perform other functions necessary for proper conference behavior(decoding streams from other participants, user events handling, independent applications working e.g. audio tool). A few different approaches to this problem were implemented and tested but so far the best one was based on decoding time.

After decoding a frame from conference participant decoding time was measured and based on that time, number of participants and experimental chosen coefficient the time between encoding consecutive frames was calculated. However this solution works fine only if encoding capabilities of conference participants are similar. If one participant’s frame rate is much higher than others, their time gaps are not long enough to deal with incoming frames. It is predicted to improve this method by adding to the calculations frame rate of other participants.

Another implemented solution was based on acknowledgments of decoding frame that were send back to the video sender. Only after he obtained all the acknowledgments(from all participants) he continued to capture video. Certainly it slowed down the sending process but it had very convenient advantage of autoregulating. This solution worked fine with OpenDVE environment but since NCS application data exchange is based on raw UDP protocol some of acknowledgment messages were lost what led to all applications hung up after some time. Of course it was possible to improving this protocol to be more reliable but it would cause further slowing down below acceptable level.

### Video format conversions

H.261 and H.263 require YUV 411 video format as its input. However, both SGI video capture library and  PC capture library do not have capabilities to capture video data in this format. SGI format is very similar – YUV 422 so conversion is very simple, although PC provides raw RGB format requires conversion process described in chapter 2 of this report. Such conversion procedures were implemented and successfully used in Video Tool.

## Audio Tool

Audio Tool is application created to enable voice communication between conference participants. It was designed only for the speech signal, however it is possible to transfer also low bandwidth music. Since application was intend for Internet, high sound quality was sacrificed for the bandwidth efficiency. Audio Tool adopts two compression options:

- GSM  06.10 - used in digital cellular telephony, very bandwidth efficient but designed just for the speech transmission.

- Intel/DVI ADPCM - takes more bandwidth but offers much better quality, good enough even for the music.

So far both option exchange 8000 samples per second sound, 16bits sound. Audio stream is coded and sent in the 100 ms samples chunks. Audio formats are automatically recognized and appropriate decompression method is chosen.

### Multiplexing

On SGI Indy multiplexing audio streams is performed using internal SGI audio library mechanisms. For each stream dedicated audio port is opened. Receiving audio data, sender ID is detected and audio chunk is decoded and put to the appropriate port.

Most PC sound card does not support multiplexing data streams. Moreover usually they support only half-duplex audio mode. To handle this situation Audio Tool implemented on PC present completely different approach. First of all special mechanism switching between sending and receiving mode was created. Initially Audio Tool is in sending mode until the moment when level of audio received from a participant is above some threshold. This threshold can be set manually by user through application graphic interface. After that, application is switched into the receive mode and participant that sent the switching packet is set to the “current sender”. Till next switch to the send mode all audio packets from different participants are ignored. Application is switched back when received audio level drops below threshold for fixed amount of time.

It allows to avoid necessity  of audio multiplexing. Of course it influences conference quality but providing it is led in polite, calm way  it will not be a big obstacle.

### Buffering

Also buffering is supported by internal SGI mechanism. Each audio port has it own samples queue whose length may be set by developer. Playing function blocks until there is enough room for the samples buffer.

Unfortunately PC does not have such convenient feature. Buffering is left for the developer concern. In Audio Tool the double buffering scheme is applied both for the audio capturing and audio playing. Upon receiving audio chunk is decoded and prepared to playing, but samples buffer is not sent to the speaker  until next audio packet is received. This next packet is stored in the other buffer when previous  one is being played. Similarly one buffer is compressed and sent while the other is being captured.

## Whiteboard

### Overview

Whiteboard is very convenient collaborative tool that allows to share drawing among conference participants. It is possible to perform following operations:

- drawing lines and basic geometrical shapes

- choosing drawing colors

- choosing line width

- displaying text

- saving and retrieving drawing from a file

### Implementation

Each drawing element is stored in memory as an “graphic object”. Object consists of following informations:

- object type

- object position

- additional location

Application creates and manages list of objects that can be also save and retrieved from a file. Each object type has its own drawing procedure. Objects are exchanged between applications using NCS mechanisms. Objects received are handled the same way as obtain as a user input.

So far whiteboard was implemented only on SGI workstations based on Motif library. However porting it on PC platform is predicted, as well as improving application of clipboard capabilities.

# NCS-Tango integration

Tango is another NPAC collaboratory system. However, Tango is more Web-oriented and does not support real-time applications. Since it was necessary to provide collaboratory environment with functionality of both Tango and NCS these two system were integrated. Because NCS works in this system in slave mode – all the conference management is performed by Tango -- conversion of NCS could be done without knowledge of Tango architecture. Thus this architecture will no be presented in this report.

To obtain seamless integration following activities were undertaken:

- removing existing graphical user interface

- agreeing message exchange protocol with Tango developers

- installing Tango communication unit instead of GUI

- Figure 5 (a) NCS-user interaction (b) NCS-Tango integration

# Future work

System created during internship provides basic set of collaboratory mechanisms, however they are far from perfection. Apart from user-feedback and handling unexpected situation improvements there are still much more important problems such as: more reliable control protocol, video rate control and audio quality. Other projects for the future are:

- adding database support to save and restore conferences from files

- porting the system into multiplatform Java environment and Web

Video On Demand project

Chapter

4

# Overview

Video On Demand system allows remotely retrieve content of movie database having full control over transmission process. From the user point of view it works as VCR with capabilities of choosing the movie from wide system offer.  VOD systems open new possibilities in education, entertainment and business applications.

# Architecture

VOD system consists of three major parts:

- Stream server - provides the stream of data from one of the server’s files, allows to start and stop transmission at any time and also to random access the data

- Database - provides the information about the movies available. It includes server address, file path, file length, frame rate, frames offsets and others

- Clients - reads data from the server. It is the client responsibility to process the data from server in proper way based on information from database

VOD system architecture is shown on figure 6.

Communication between database and clients is performed via Netscape Navigator using standard HTTP, CGI mechanisms. Data from the server are transmitted using socket connections (TCP). Transmission control is done by additional control connection based on VOD client-server protocol.

- Figure 6 VOD System architecture

# Audio-Video H.263 Client

AV client is implemented for the SGI Indy, IRIX 5.x platform. It uses H.263 video compression, and either GSM or ADPCM audio compression. It supports QCIF and CIF file formats. It allows to play the movie, stop it and random access the movie.

### New elements

In existing movie formats audio and video data were included in the same stream. But what we wanted to do was to synchronize AV data from two independent streams. Hence it was necessary to open two server connections, send and receive control messages from two separate channels. Unfortunately database system offers information just about video stream. Therefore audio configuration had to be included in the data file. It was done by adding special audio header whose structure is presented below in table1

- Table 2 Audio header

| Name | Length (in bytes) | Description |
| --- | --- | --- |
| Title | 12 | String “npac-audio”. Indicates that file is NPAC audio stream |
| Rate | 2 | Rate in samples per second |
| Channels | 1 | Channels number: 1 - for mono 2 - for stereo |
| Sample width | 2 | Sample with in bits |
| Code format | 10 | String indicating compression type “adpcm”, “gsm” supported |

### Synchronization

Synchronization is based on the internal SGI Audio Library mechanisms. Procedure sending audio samples to the speaker port blocks until all previous sent samples are played. In this case it is enough to send audio portion and then decode video frame to achieve the synchronization and keep the frame rate. The only problem that this requires single video frame decoding which moves the responsibility of single frame reading from the decoder to the AV client.

To obtain this effect  some changes were necessary both at the decoder and client side:

- the original Telenor’s H.263 decoder was transformed into library that allows decoder initialize and then decode single frame passed as function argument

- special function detecting H.263 frame end was implemented and use in single frame reading procedure

- to make processes of reading data from socket and taking the single H.263 frame independent a system of ring buffers was implemented

### Random access

Since audio portions take always the same size it isn’t a problem to find beginning of the portion knowing its size. Situation differs in H.263 video compression. First, H.263 frames have different length, second because it is predictive compression only INTRA frames can be accessed in this way. The way of solving this problem is described in chapter H.263 stream offsetting.

### stream offsetting

H.263 system is supposed to be used as a preview tool to the MPEG movies. Hence H.263 sequence will be acquired by converting MPEG files. As a preview tool it is absolutely necessary to have the capabilities of random access. It implies necessity of offset files generation.

To obtain offset files it was necessary to add some new features to the H.263 encoder:

- possibility of encoding INTRA frame every fixed number of frames (only INTRA frames can be accessed randomly)

- writing offset file with number and stream position of every INTRA frame

- This mechanism allow to generate offset file parallel with movie converting

# Future work

Since H.263 AV client seems very good candidate for preview tool for NPAC VOD system it can be improved in several ways:

- implementing multicast client what enables multiuser video system without delays and high network traffic

- improving client (and server) stability

- creation easy to use MPEG->H.263 conversion tools

Reference

RETTINGER, LEIGH ANNE “Desktop Videoconferencing: Technology and Use for Remote Seminar Delivery” (Under the direction of Dr. Thomas K. Miller III.), 1995, http://www2.ncsu.edu/eos/service/ece/project/succeed_info/larettin/thesis/

Degener, JUTTA “Putting the GSM 06.10 RPE-LTP algorithm to work”, December 1994, http://www.ddj.com/ddj/1994/1994.12/degener.htm

HUNG, ANDY C. “ PVRG-P64 Codec 1.1”, November 1993

DRAFT ITU-T Recommendation H.263, July 1995, http://www.fou.telenor.no/brukere/DVC/h263_wht/

TELENOR “H.263 Advanced Negotiable Options”, December 1995, http://www.fou.telenor.no/brukere/DVC/h263_options.html

BULAWA, JANUSZ “Integration of multimedia collaboratory environment with Web browser” – Master Thesis, September 1996

DRAFT ITU-T Recommendation T.120 “Data Protocols for Multimedia Conferencing”

INSOFT INC. “OpenDVE Architectural Overview”, 1996

VTEL Corporation  “H.320: A Quality Requirement Guide”, 1996