Development of Integrated, Web-based Videoconferencing System

Northeast Parallel Architectures Center

Internship Report

Tomasz Stachowiak

Syracuse 1997

Table of contents

INTRODUCTION

Background

Overview

TECHNOLOGY

Video

Video formats

Video compression

Introduction

Audio

Digital audio

Audio compression

ADPCM

GSM 06.10

Desktop videoconferencing systems

ITU-T H.320

Picture Resolution

Frame Rate

Pre and Post Processing

Motion compensation

Audio consideration

Data rates

Existing Internet conference products

NPAC CONFERENCING SYSTEM

Overview

Conference management

Conference participants

Conference applications

Architecture

Conference Engine

NCS API

Session API

Application API

Conference Manager

NCS Server

Applications

Video Tool

codec improvement

Frame rate control

Video format conversions

Audio Tool

Multiplexing

Buffering

Whiteboard

Overview

Implementation

NCS-Tango integration

Future work

VIDEO-ON-DEMAND PROJECT

Overview

Architecture

Audio-Video H.263 Client

New elements

Synchronization

Random access

stream offsetting

Future work

Introduction

Chapter

1

# Background

This report describes the work carried out during the internship in Northeast Parallel Architecture Center at Syracuse University. This internship was a partial fulfillment of requirements for the Master Degree program of Franco-Polish School Of New Information And Telecommunication Technologies (EFP) in Poznan, Poland.

Internship took place between July and December 1996 under supervision of Dr. Marek Podgorny, NPAC’s Associate Director.  The internship program was a joint venture between EFP and NPAC and was conceived and set up by Marek Podgorny and EFP’s  Professor Dr. Czeslaw Jedrzejek.

# Overview of NPAC

The Northeast Parallel Architectures Center is a research and development center focusing on high performance computing and communications. Established at Syracuse University in 1987, the center is directed by Geoffrey C. Fox, a pioneer in the development and application of parallel computers, who assumed NPAC leadership in 1990. Since then the scope of NPAC has broadened to include world class computational science research and education, and most recently research and development in high performance communications with particular focus on support for the National Information Infrastructure (NII).

There is a strong emphasis on computational science applications where one seeks to obtain 'real solutions to real problems'. NPAC's InfoMall technology transfer program puts high-performance computing and ommunications (HPCC) to work in industry. Other major projects include esearch and development in the areas of parallel languages and parallel ompilers, including work on High Performance Fortran (HPF) – a tandardized parallel version of Fortran; distributed computing; parallel atabase technology; integration of relational databases with the NII; arallel algorithms; distributed and cluster computing; and network digital multimedia applications providing Information, Video, Imagery, and Simulation on demand. Our major research and development programs naturally strengthen the new educational program in computational science.

# Address of NPAC

111 College Place

Syracuse University

Syracuse, New York 13244-4100, USA

phone: (315) 443 17 22

fax: (315) 443 19 73

# Contact to Supervisor

Marek Podgorny, Ph.D.

Northeast Parallel Architectures Center

111 College Place

Syracuse University

Syracuse, NY 13244-4100, USA

e-mail: marek@npac.syr.edu

# Overview

Report presents the results of the internship, which consisted of two projects:

- NPAC Conferencing System - design and implementation of a multiplatform, web-based, desktop videoconferencing system supporting a variety of collaborative tools (e.g. Video Tool, Audio Tool, Whiteboard)

- Video On Demand - implementation of an Audio-Video client for existing NPAC VOD architecture, using latest video and audio technology (e.g. H.263 video codec)

First part of this report briefly describes technologies used in development of both projects. Existing software products performing similar tasks are reviewed. Second part describes details of the design and implementation work.

Both projects utilized state-of-the-art technologies, some of them still evolving. As the project progressed, its goals have been modified few times to keep the pace with changing development environment and end-user requirements.  While this dynamic situation complicated software development, it has however resulted in a set of implementations that are state-of-the-art. We will continue to use these implementations in the next phase of the project in which the component technologies being developed in NPAC are integrated into one generalized collaborative environment.

Technology

Chapter

2

# Video

## Color space representation

To represent any color distinguished by a human eye three primary components are necessary. This phenomenon is widely used in television and video technology. By mixing three color components: red, green and blue (RGB) it is possible to display all other colors. In RGB representation, certain number of bits is assigned to represent each color component. The most demanding applications call for 8-bit representation for each R, G, and B, for a total of 24 bits (3 bytes) per pixel. This representation is known as “True Color”.

Color images and videos are notoriously demanding in their storage requirements. For practical networked applications images and video must be compressed. We will discuss basics of the image and video compression later.  Physiology of human eye suggests that, as a first step towards compression, a more efficient color representation is possible by isolating the luminance signal responsible for brightness and the two chrominance components describing colors. Since human eye is less sensitive to color than to brightness changes, it is possible to reduce amount of information in chrominance components without affecting the image quality. Another advantage of this format is concentration of the image information in the luminance component, which results in reduced elements correlation. Hence components can be compressed separately without much efficiency loss. Image size reduction is achieved by subsampling chrominance components at the rate of 2, 4, or even 16 times lower than the luminance component.

Television standards use different types of color spaces. In European’s PAL and SECAM,  YUV color space is used, while YIQ representation is used by NTSC standard. In both representations the Y component (luminance) is the same. For digital applications, the equivalent of YUV is YCbCr where Cb and Cr correspond to U and V components respectively. This representation allows normalization of color components ranges. If luminance range is between <0; 1> the UV range is undefined but Cb and Cr ranges are between <-0.5; 0.5>.

The conversion between RGB and YCbCr formats can be represented by the following matrix transform:

YCbCr=0.2990.5870.114−0.1687−0.33130.50.5−0.4187−0.0813RGB

RGB=101.4021−0.34414−0.7141411.7720YCbCr

Additionally, H.261, H.263 and MPEG conversions perform slightly different scaling. According to CCIR Recommendation 601, color space is re-scaled and shifted according to the following formula: Y’ = 219/255 * Y + 16, Cb’  = 224/255 * Cb + 128, Cr’ = 224/255 * Cr + 128.

## Video compression

Uncompressed digital image is usually too large to effectively store, treat or transmit through the network. Many compression techniques have been devised to compress digital video. Requirements for an efficient video codec are very demanding - it must have high compression ratio, keep the image quality and be fast. Additionally, lossless  compression methods such as Huffman, Arithmetic Coding or LZW do not work well on the images. It was hence necessary to create compression techniques designed specially for video. At present, the most popular algorithms for low bandwidth video compression are based on the ITU H.261 recommendation.

H.261 compression uses two different techniques enabling effective video compression:

- spatial redundancy removal – both INTRA and INTER frames coding

- temporal redundancy removal – INTER frames coding

## Spatial redundancy removal

- Figure 1 INTRA frame coding

Spatial redundancy removal is method based on the Discrete Cosine Transform (DCT). YUV image is divided into blocks 8x8 pixels each. DCT is performed on each block according to the following equations:

F[u,v]=14i,jL(u)L(v)cos(2i+1)up16cos(2j+1)vp16f(i,j)

L(x)=12forx=01otherwise

After DCT, subsequent block values represent information for increasing signal frequency. Frequency coefficients could be also obtained using FFT (Fast Fourier Transform) but FFT is slightly less stable as a computational method.

Since human eye is less sensitive to higher frequencies, it is possible to eliminate them without losing much of the image quality. Such elimination, or actually reduction, of the component information is achieved during the coefficient quantization step. Each block value is divided by the proper constant collected in the quantization table. There are different quantization tables for luminance and chrominance signals. Scaling quantization tables by certain factor increases or decreases image quality. Additionally, custom quantization tables can be used and put into the image header.

The next step of the compression process is the Zig-Zag scan. It converts a two-dimensional 8x8 block into a 1x64 vector, grouping low frequencies in top of the vector. On such a prepared vector the standard RLE (Run Length Encoding) and Huffman compression algorithms are performed.

- Figure 2 Zig-Zag Scan

#### Temporal redundancy removal

Temporal redundancy removal technique uses the correlation between two consecutive frames in video stream to reduce their compressed size. To achieve this goal, the encoded H.261 stream consists of two types of video frames:

- INTRA (I-frames) -- coded only with spatial redundancy removal technique

- INTER (P-frames) – “predicted”, coded based on the “pseudo-difference” from the previous frame

The first stage of INTER frame coding is the motion vector estimation process. For each macroblock in coding frame (macroblock is a structure consisting of four luminance blocks and two chrominance blocks – Cr and Cb) the area of the previous frame (reference frame) that matches it best is found. Then, instead of coding the macroblock, it is enough to code the difference between the best match area and add the information about motion vector to the compressed stream. Such information can be put into the macroblock header. The difference is then coded using spatial redundancy removal, the only difference being separate quantization tables for INTER frames.

#### Bitstream structure

As we mentioned above, to efficiently compress a video stream, apart from the video signal data we need also some additional information. Thus, it was necessary to create an H.261 video stream structure to enable efficient information retrieval by the decoder. Simplified scheme of the H.261 bitstream structure with brief description of its fields is presented below.

- Figure 3 H.261 Bitstream structure

| Field | Description |
| --- | --- |
| PSC | Picture Start Code, unique sequence to delineate boundaries between pictures |
| TR | Temporal Reference, timestamp used for audio synchronization |
| Ptype | Picture type (INTRA / INTER) |
| GOB | Group of Blocks |
| Grp # | Group number, enables skipping the whole group indicating its number |
| Gquant | Group Quantization Value - common value used for quantization of whole group |
| Addr | Address of macroblock in case it is exact match |
| Type | If good match cannot be found block may be coded in INTRA mode |
| Quant | Quantization Value for macroblock |
| CBP | Coded Block Pattern, bitmask indicating which blocks are present in case they matched poorly |

#### Motion vector search

Motion vector estimation is the most computational intensive part of H.261 encoding process. Hence, it is very important to implement an effective method of search. So far multiple algorithms were used for this purpose. Usually faster method means lower video quality.  Two most important motion estimation algorithms are:

- full spiral search -- checks all the positions in searching area starting from the middle and moving spirally outward

- two-dimensional logarithmic search – similar to binary search, checks only a few locations in the area then continues searching around the location that matched best. Logarithmic search is the least computational  intensive, however it does not provide highest quality

H.263 is an improved version of H.261 compression algorithm. The idea of image coding is the same but some additional elements were added which significantly increased the compression efficiency. At the same percepted image quality it was possible to reduce bit stream size by 30-50%. The main factors that enabled this significant progress are:

- half pixel motion vector prediction negotiable options

- improved (VLC) tables

#### Negotiable options

Encoder can switch on these options if decoder signals that it is capable to decode them. Encoders should include all of them because each one can significantly increase the effectiveness.

- Unrestricted Motion Vector (UMV) mode. In this mode motion vectors are allowed to point outside the picture. The edge pixels are used as prediction for the "not existing" pixels. With this mode a significant gain is achieved if there is movement along the edge of the pictures, especially for the smaller picture formats. Additionally, this mode includes an extension of the motion vector range so that larger motion vectors can be used. This is especially useful in case of camera movement.

- Advanced Prediction mode. This option means that overlapped block motion compensation is used for the P-frames. Four 8x8 vectors instead of one 16x16 vector are used for some of the macroblocks in the picture, and motion vectors are allowed to point outside the picture as in the UMV mode above. The encoder has to decide which type of vectors to use. Four vectors use more bits, but give better prediction. The use of this mode generally gives a considerable improvement, especially subjectively because OBMC results in less blocking artifacts.

- Syntax-based arithmetic coding mode. In this mode arithmetic coding is used instead of VLC coding. The SNR and reconstructed frames will be the same, but generally fewer bits will be produced. The average gain for inter frames is 3-4%. This gain depends on the sequence, the bitrate and other options used. For intra blocks and frames, the gain is higher, on average about 10%.

- PB-frames mode. A PB-frame consists of two pictures being coded as one unit. The name PB comes from the name of picture types in MPEG where there are P-pictures and B-pictures. Thus a PB-frame consists of one P-picture which is predicted from the last decoded P-picture and one B-picture which is predicted from both the last decoded P-picture and the P-picture currently being decoded. This last picture is called a B-picture, because parts of it may be bidirectionally predicted from the past and future P-pictures. For relatively simple sequences, the framerate can be doubled with this mode without increasing the bitrate much. For sequences with a lot of motion, PB-frames do not work as well as B-pictures in MPEG. This is because there are no separate bidirectional vectors in H.263, the forward vectors for the P-picture is scaled and added to a small delta-vector. The advantage over MPEG is much less overhead for the B-picture part, which is really useful for the low bitrates and relatively simple sequences most often generated by videophones.

# Audio

## Digital audio

Frequencies perceived by human ear as sounds are typically between 20 Hz and 20 kHz. Human voice can produce frequencies between 40 Hz and 4 kHz. Sounds to be digitized must be converted from the continuous time and value domain into the discrete one.

Process of converting analog sound in time domain is called sampling. According to Nyquist’s theorem, the signal has to be sampled with rate at least twice higher than its maximal frequency. Typically computer audio cards offer sampling rates from 8 kHz to 44.1 kHz depending on required quality. 8 kHz sampling rate is sufficient for the human voice sampling.

Then audio signal is quantized (converted to discrete values). During this process some information is lost, which results in appearance of quantization noise. Its influence on audio quality depends on the number of quantization levels. Increase of the number of quantization levels leads to necessity of coding audio sample with more bits. Usually this number varies from 8 bits (256 levels - voice quality) to 16 bits (65536 levels - CD quality).  Because human ear is not equally sensitive to signal changes at different levels, some quantization methods (e.g. PCM) are not uniform, but logarithmically spaced. It improves audio quality without changing the sample size.

## Audio compression

Transporting uncompressed digital audio takes significant bandwidth. Therefore, in most videoconferencing system various compression algorithms are used. Some of them are applicable to general audio signals, others are specifically designed for compression of human speech (Klingon’s speech compressors seem to lag behind).

### ADPCM

ADPCM compression algorithm uses the correlation between adjacent audio samples to reduce bit rate. It transmits only the differences between samples and their predicted values, which have less dynamic range than the samples themselves. Predictor coefficients and reconstruction levels are calculated dynamically using coded signal. It allows to reduce the bandwidth but make this adaptation technique more susceptible to transmission errors. There are few ADPCM standards like e.g. Intel/DVI or G.721. Intel/DVI is not very computationally intensive, and the quality of compressed audio is reasonable  good  even for  music.

### GSM 06.10

GSM is telephony standard defined by the European Telecommunications Standards Institute (ETSI). The GSM 06.10 compressor models the human-speech system with two digital filters and an initial excitation. The linear-predictive short-term filter, which is the first stage of compression and the last during decompression, assumes the role of the vocal and nasal tract. It is excited by the output of a long-term predictive (LTP) filter that turns its input - the residual pulse excitation (RPE) - into a mixture of glottal wave and voiceless noise. GSM encoder compress 160 16-bit voice samples into 264-bit GSM frame. GSM 06.10 is faster than codebook lookup algorithms such as CELP. It offers 13 Kbps bandwidth.

# Desktop videoconferencing systems

Desktop videoconferencing is interesting and much less expensive alternative for “room” based videoconferencing systems. In desktop videoconferencing,  the participants use  their computers similarly as they use telephones. From business point of view it has many advantages: reduce high travel costs, speed up communication, increase productivity. However, nature of video data requires high performance equipment and high bandwidth networks what is certainly connected with high expenses. Unfortunately, there are also other drawbacks: current systems interoperability problems, and limited availability of transmission lines.

To provide potential users with a set of requirements which desktop videoconferencing system meets a few videoconferencing standards were created. The most important is ITU-T H.320 standard to which most videoconferencing systems are compliant. This chapter briefly describes videoconferencing standard H.320, and provides a brief review of existing conference systems for the Internet.

## ITU-T H.320

H.320 is the ITU-T standard for videoconferencing. As the ITU-T develops standards, one goal is to define the minimum requirements all videoconferencing vendors must support. This minimum requirement ensures that all H.320-complaint systems will communicate with each other. The optional requirements affect the quality of the audio and video and their implementation is left to the manufacturer.

A videoconferencing system can belong to one of the following classes:

- Class1 - minimum level of support (required by H.320)

- Class 2 - support of some optional features

- Class 3 - support of all optional features

### Picture Resolution

In H.320 system there are two types of picture resolutions: Quarter Common Intermediate Format (QCIF) and Common Intermediate Format (CIF). Class 1 systems will only support QCIF, Class 2 systems may support CIF, and Class 3 will support CIF. The ability to support CIF resolution is critical to the quality of the picture. CIF capable systems provide a picture resolution of 352 x 288. QCIF provides quarter the resolution -- 176 x 144. A QCIF picture will look like an unfocused blocky image. A high-quality system will be forced to degrade its resolution if it connects to a low-quality system. For example, if a Class 1 and Class 3 systems connect in a call, the Class 3 system will be forced to display a QCIF resolution.

### Frame Rate

Frame Rate, also referred to as the Number of Frames Per Second (fps), is the number of times the picture image is refreshed. H.320 systems can support frame rates of 7.5, 10, 15, or 30 fps. A low frame rate will cause the motion to appear jerky. The higher the frame rate, the smoother the motion will be. Class 1 systems are required to support a frame rate of 7.5. Support for the higher frame rates are left to the manufacturers and the ones with a desire to deliver high quality products will support higher frame rates. Typically Class 2 systems will support up to 15 fps. Class 3 will support up to 30 fps. Unfortunately even the manufacturers who deliver Class 3 systems are affected by manufacturers who are not driven to produce high quality products. This means if a Class 1 system connects to a Class 3, the Class 3 system will have to downgrade its capabilities to communicate to the Class 1system. The Class 3 system would be forced to use 7.5 fps for the call to work.

### Pre and Post Processing

Pre-processing is a complex process to reduce the amount of re-coding the background. This is not required for Class 1 systems. If pre-processing is not used the video encoder can spend a lot of time encoding 'noise' caused by the poor lighting of the camera. These situations may make the system think that there is motion in the background when in fact there is none. Pre-processing ensures that only real motion gets encoded. Post-processing will compensate for picture degradation caused by fast motion. Post-processing can reduce the blocking and noisy effects caused by H.320 video coders. Post-processing can also be used to enhance the frame rate and reduce the jerky motion effect caused by low frame rates.

### Motion compensation

There are two aspects of motion compensation: motion estimation and the actual motion compensation. Motion estimation is performed at the encoder to determine what the motion vector should be. Motion compensation is performed at both the encoder and the decoder, and consists of moving blocks of data around based on the motion vector. Essentially, motion compensation only encodes the section of video where motion has occurred rather then encoding the entire video area for every frame. This is especially important at lower bit rates. All H.320 systems must have the ability to decode a motion compensation signal. Providing encoded motion compensation is optional. The encoding process is where video quality improvements are made. A Class 1 system will not support the encode compensation, a Class 2 system may support a limited form of motion compensation, and a Class 3 system will support a higher form of encode motion compensation.

### Audio consideration

H.320 specifies three types of audio:

- G.711 -- 48-64 Kbps narrowband

- G.722 -- 48-64 Kbps wideband

- G.728 -- 16 Kbps narrowband

A Class 1 system only has to support G.711. G.711 is telephone quality audio (narrow-band, 3KHz). G.722 produces stereo-quality audio (wideband, 7KHz). A higher data rate, typically 256 Kbps and above, Class 3 systems will support G.722 offering the user the best audio quality available. Class 1 systems will not support G.722, and Class 2 typically will support G.722. G.728 (16 Kbps) is narrow-band audio but it is important for low bit-rate calls, typically less then 256 Kbps. For example, in a 128 Kbps videoconference (typically via BRI) a high-quality manufacturer will offer the user to select between G.722, 64 Kbps, or G.728, 16 Kbps. If audio quality is more important then video quality (64 Kbps for the audio will only leave 64 Kbps for the video), the user can choose to use G.722. If video quality is more important the user will use G.728. The audio will use 16 Kbps leaving 112 Kbps for the video. High-quality manufacturers will allow the user this choice, not dictate it to them. Class 3 systems again are limited by the lesser Class systems. If a Class 3 system establishes a call with a Class 1 system the audio will be G.711. The Class 3 system is forced to use lower-quality audio in order for the call to work.

### Data rates

Data rates have an impact on the perceived quality. The higher the data rate the better the video quality will be. At T1 speeds (1536 Kbps) the video quality will be optimal. Realistically, most users will not have network access at this speed. This is mainly a cost issue. In today's market, many users are using 768 Kbps as a trade-off between high quality video and cost. 384 Kbps  (H0) is probably the most common data rate in use today, 128 Kbps is becoming more popular with the availability of ISDN. As desktop videoconferencing becomes more popular, 128 Kbps bitrate will become one of the more common data rates.

Many factors affect the overall quality of an H.320 system. The ability of vendors to implement optional features will greatly impact the overall quality. The user should understand their environments and the application the H.320 system will be used in. These factors can help make a decision on what the requirements for the video conferencing system will be. Applications can range from desktops to boardrooms and each will have its own requirement. Not only will the system the user purchases determine the quality but also the capabilities of the system on the other end will have a major impact to the overall quality. A less capable system will always force the high quality system to downgrade its quality in order for the conference to be held. The best solution for a user is to require a system that is flexible and software upgradeable. Flexibility means the system can be configured to the requirements of the user, which may change over time or even may change from conference to conference. Software upgradeability provides investment protection to the user by allowing the user to take advantage of new features without being required to buy new hardware.

- Table 1 H.320 conformance test

| Item | Class 1 | Class 2 | Class 3 |
| --- | --- | --- | --- |
| Picture Resolution | QCIF mandatory | CIF optional | CIF mandatory |
| Frame Rate | 7.5 fps | up to 15 fps | up to 30 fps |
| Motion Compensation | must be able to decode motion compensation signal | limited form of encode motion compensation | higher form of encode motion compensation |
| Audio | G.711 | G.722 | G.722, G.728 |

## Existing Internet conference products

The teleconferencing systems for the World Wide Web environment differ significantly from the commercial ones. The major reason is that here one can not assume a large network bandwidth available. This in turn impacts a video coding scheme, a picture size and a number of transmitted frames per second. Usually these systems are fully implemented in software and use more sophisticated video coding schemes in order to cope with insufficient network bandwidth available. There also has been a lot of work done in order to develop appropriate transport protocols (see section 5.4) which would be able to transfer the increasingly demanding amount of data in videoconferencing. As a result, an entire suite of protocols and applications has been developed which seeks to alleviate the many problems that exist in video conferencing.

In the World Wide Web environment these are a multicast transport schemes and the real-time transport protocols (such as RTP, described in section 5.4.1) that enable a real-time multi-point multimedia conferencing. Along with the development of the MBone and multicast protocols (see section 5.4.3) it became feasible to run conferencing applications on the Internet. All conferencing tools for WWW use those technologies without which they would simply not be operational.

In the following paragraphs we describe the most popular teleconferencing packages currently in use.

Network Video (NV), developed by Xerox Park was one of the first network video only conferencing packages. The entire system is implemented in software and makes use of RTP and IP multicasting. This package uses a proprietary video-coding scheme, which gives a compression factor of about 20-35. The encoding of a one frame takes about tens of milliseconds and typical framerate equals to 10-15 frames per second (reasonable for a local area network with a typical workstation). The Haar wavelet transform and discrete cosine transform are used to encode those 8x8 pixel blocks, which have changed significantly as compared to the previous frame. No motion estimation is employed. A user can increase a framerate by lowering the quality of coded pictures. This is mainly imposed by the network's inability to handle a large amount of video data. NV is capable of video capturing from an X11 window and therefore making itself compatible with many platform-dependent video grabbers. For more details on NV, please refer to [19] or [20].

Inria Videoconferencing System (IVS), developed by INRIA researchers was one of the first video and audio conferencing systems. The audio coding scheme in IVS provides a quite good quality and generated bitstream takes up to 64 Kbps. Video data uses a mere 128 Kbps. The images are usually a quarter screen or smaller. IVS uses H.261 video compression algorithm, which due to its advanced motion estimation and compensation techniques reduces significantly the amount of data to be transmitted. The drawback is that due to its complexity H.261 encoding and decoding takes more time. Therefore, on a lower end its performance in video encoding is rather not sufficient. On the other hand, IVS system is the only one available supporting synchronized audio and video in one application. The popularity of IVS package may grow with the appearance of the faster workstations. For more details on IVS, please refer to [19] or [20].

Videoconferencing Tool (Vic), developed by Lawrence Berkeley Laboratories, is perhaps the most flexible conferencing system available. It combines all features that were pioneered by its predecessors, NV and IVS, along with some new features specifically designed for Vic. It supports many more compression algorithms and has a very good user interface. It is a video only tool and has to be used in conjunction with an audio conferencing tool VAT (Visual Audio Tool) and session management tool SD (Session Directory). Vic supports the following video compression schemes: MPEG, JPEG, NV, CU-SeeMe, CellB and its proprietary intra-H.261. Therefore, using the Vic tool, one can join many different conferences as long as they use one of the aforementioned compression algorithms. The innovation here is that instead of encoding the block changes in a picture, which depends on previous frames, this version of H.261 simply encodes the entire changed block. Vic takes advantage of the RTP protocol to coordinate many different compression schemes (RTP header can have any application specific extensions). A new feature called conference bus is implemented in VIC. This allows all participants to receive critical information concerning a course of a teleconference (floor assignments, different conference formats). Another big advantage of VIC is that it is able to support different hardware codecs. Vic reaches a sending framerate of 30 fps at a bandwidth of 1.1 Mbps. Unlike NV, VIC is not able to capture video data directly from an X11 window and therefore is dependent on a video grabber type in use (Vic supports a very limited set of video grabbers). For more details on Vic, please refer to [19], [20] or [8].

NPAC Conferencing System

Chapter

3

# Overview

The goals of this project are:

- create infrastructure for collaborative, distributed, real-time applications (video, audio)

- implement multiplatform, modular conferencing system for Internet, compliant to existing standards (T.120, H.320)

- create development environment for improving and adding collaborative components

Initially such system was developed in NPAC based on InSoft's OpenDVE software development kit. Since there have been numerous restrictions and limitations to OpenDVE-based conferencing system, we decided to develop our own conferencing software infrastructure. Reasons for creation NPAC Conferencing System (NCS), independent of OpenDVE:

- full control over data transmission process – especially important for real-time applications(video, audio)

- ability to porting the system on different platforms

- ability to create system compliant to conference standards(T.120, H.320)

- possibility of implementing standard real-time data transportation protocols(RTP, RSVP)

- ability to integrate NCS seamlessly with other NPAC projects(e.g. Tango)

- integrate the multimedia conferencing tools into the World Wide Web environment

First, NCS was implemented on SGI Indy workstation. However, after satisfactory performance has been achieved performance under IRIX 5.X operation system, NCS was ported successfully to the PC Windows NT/95 platform.

# Conference management

A conference refers to a group of geographically dispersed nodes that are joined together and that are capable of exchanging audio and video information across various communication networks. Conference participants may have access to various types of media handling capabilities such as audio only (telephony), audio and data, audio and video, or audio, video and data.

The conference in NCS is based on the notion of a session. Session is a group of users on different machines connected in virtual many-to-many channel. So far, there can only be one session on one machine and just one user on the machine can participate in conference. Many applications can be attached to one session.

## Conference participants

Conference participant can start new session and invite others. Invitation will be visible for remote user provided he has his Conference Manager running. He can also enable the Conference Engine “watchdog” function, which automatically starts Conference Manager and displays incoming invitation. Remote user can accept or reject the invitation. If he accepts invitation he will become conference participant. From this moment all his NCS application will collaborate within the current session. Each conference participant is free to invite users to participate in conference. So far, there is no other way to join the conference, i.e., a non-invited user cannot join an existing conference call. The system acts very much as phone system. This behavior is restrictive and will be changed in a future release.

Each participant has his own user identification number, unique within the session. Every conference message includes this user id. There is always one special conference participant – state-keeper. Although all participants hold all the information about the conference, it is necessary to select one participant that provides information to new participants and resolve any state conflicts. Usually, it is the conference initiator but in some situations (e.g. when initiator left) it may be another conference member.

## Conference applications

Conference application is every application, which is attached to Conference Engine and collaborates within the session. Each participant can start conference application. Applications can be started from Conference Manager control panel or independently. If application is invoked from Conference Manager panel it will be started automatically on all participant machines. Also, exiting the application from Conference Manager panel causes closing all participant applications. If application is started independently Conference Engine may reject the application attempt to connect to the session.

# Architecture

The main assumption during NCS development was modularity. Modular system structure enables easy and quick modification and improvement of some of its parts without affecting the others. Based on core modules it is possible to add new collaborative applications using existing NCS mechanisms. It is especially important in Internet environment where system context evolves dynamically.

NPAC Conferencing System consists of two main modules. The first module is the Conference Engine (CE) which contacts the remote hosts, processes control messages from them and passes the indications of events to the Conference Manager (CM). Conference Engine also processes the local messages from Conference Manager. To create Conference Manager the set of API functions is provided. There is also an API for the conference application management.

NCS uses UDP (User Datagram Protocol) to speed up data exchange. For real-time applications UDP fulfills the performance requirements without affecting quality of service. It is up to application to create higher level, more reliable protocol if necessary. However, implementing such a protocol for priority messages is also planned as internal part of the Conference Engine.

- Figure 4 NCS Architecture

## Conference Engine

Conference Engine (CE) is the core of NCS system. It performs the following functions:

- keeps the information about conference members and applications

- receives the requests and sends the indications to Conference Manager and conference applications

- exchange the control messages with other conference members

CE sends and receives all the conference control messages. However application data exchange is performed independently by the applications. This solution allows avoiding bottleneck in CE, which would act as a message router for all NCS messages in otherwise. This is particularly important for real-time applications for which the system was designed. Therefore, applications keep copies of some necessary conference information (e.g. members’ addresses) and they are informed of any changes in that data by the CE.

Conference Engines on different hosts communicate with the UDP socket connections. Communication with CM and local applications is supported by the same type of connection and even by the same socket. To accomplish this, a special kind of control protocol was designed consisting information about message sender, receiver and type.

## NCS API

NCS API was created to facilitate development of NCS-based applications. Conference Engine and API provides the backbone for conferencing system. It allows flexible improvement of NCS, creation of a variety of new applications and enrichment of its functionality. According to the type of application that is implemented, there are two different kinds of API functions:

- session API - for development of conference management applications (Conference Manager)

- application API - for development of ordinary conference applications (Video Tool, Whiteboard)

API functions perform all the message exchange and offer easy a straightforward callbacks mechanism to proliferate  NCS events. They also enable acquisition of all kinds of conference information that may be useful for application.

### Session API

In the first step, the conference manager application must initialize conference with ncsSessionInitialize()  function which returns error value upon initialization failure. Initialization opens the control socket and informs the Conference Engine that managing application is active.

Then, the conference callbacks must be set with ncsSessionAddCallback() function. Callbacks are a simple and convenient mechanism to inform applications of conference events. Developer writes the procedures handling events and adds them as callbacks to the conference system.

To properly react to system events, application developer must implement the checking loop. Depending on developing platform, it may be either an X Windows working procedure, Windows thread, or ordinary loop. Inside the loop one can obtain the application file descriptors set, which may be then used as a parameter for the standard socket select() command. If at least one of the file descriptors is active, a function processing incoming messages (ncsProcessIncoming()) should be invoked.

After that, the application may wait for invitation (which invokes a callback function) or start a new session (ncsSessionCreate()) and invite participants (ncsSessionAddMember()). NCS session API also provides several other procedures dealing with invitation handling, applications management, and application information retrieval.

### Application API

To collaborate within the current session, each application must attach to the Conference Engine with ncsAppAttach() command, which returns the application handle used by all other conference operations. Application API has a similar callback mechanism to session API. It also handles the incoming messages in a similar way. Additionally, the loop function ncsAppGetData() must be invoked to retrieve incoming messages from other participants.

Application can obtain some conference information (e.g. its id, number of participants), send message to all conference participants or to a particular participant. Application detaches from Conference Engine using a call to ncsAppDettach().

## Conference Manager

Conference Manager (CM) creates NCS user interface and interacts with user to manage the conference. It uses API session functions to:

- create and destroy session,

- invite conference members,

- accept or reject conference invitations.

Additionally, CM offers interface to start and close NCS application. Information about system application is included in an easily configurable text file, what ensures the required system flexibility.

## NCS Server

Server is an optional part of NCS – system works properly without it. However, working with a server is much more convenient, because server keeps track of all active users of the NCS system (in other words, the server provides directory services). This important information allows avoiding invitation of non-existent (from the NCS point of view) users. In the future, functionality of the server will be extended to provide other useful information about NCS users

# Applications

## Video Tool

Video Tool is an application implemented using NCS API over the backbone of Conference Engine and Conference Manager. Video Tool has the capabilities of:

- capturing, compressing and sending video stream to all conference participants

- receiving, decompressing and displaying video from other participants

Since it is the application designed for the Internet, it adopts two very efficient video compressing techniques:

- H.261 – standard codec, improved and adapted for the needs of videoconferencing

- H.263 – another standard codec, improved and adapted for the needs of videoconferencing by Janusz Bulawa

Both options provide very good picture quality. H.263 option takes less bandwidth but it is more CPU intensive, thus for slower machines H.261 algorithm is recommended. Additional feature enables only INTRA frame compression, which speeds up coding process. However, this option causes significant increase of used bandwidth.

Video Tool automatically recognizes received video format and uses appropriate decompression method. All the video frames are captured and sent in QCIF format, however it is possible to switch to the “expand mode” where QCIF frames are extrapolated to CIF format. Application includes statistics module, which provides information about bitrate, framerate and other connection parameters.

### Codec improvement

Initially, Stanford University H.261 encoder offered very poor performance. It was absolutely necessary to improve it for utilization with NCS. In motion compensation compressing algorithms the most computational intensive part is motion vector search. After extensive performance testing it turned out that motion estimation function takes 70% of compressing time. The algorithm used in Stanford codec was full spiral search, which is very simple but also inefficient. Thus we chose to change the motion estimation algorithm for a faster one. The fastest motion search algorithm is 2D logarithmic search, however it offers worse quality. Since high quality in rather static conference images is not very important and performance is the most crucial factor, we decided to use this method in our implementation. The source code for logarithmic search algorithm was taken from Berkeley MPEG encoder and adopted for our compressor. This improvement increased encoder performance almost three times.

Additionally, we enabled the H.261 encoder in “INTRA frames only” mode. This required removal of the motion estimation part and decoding reference frame part. It speeds up compression process of another 20%.

Apart from performance improvement H.261 codec required other changes to adapt it for videoconferencing purpose:

- converting executable program to a library

- retrieving the initialization procedures and encoding/decoding single frame functions

- converting i/o functions – so far they communicated only with the files, they were converted to pass the data in argument and return data as a memory pointer

- multiplexing all the reference frame information for multi-stream decoding (streams from different participants)

- adding INTRA frame every fixed number of coded frames – necessary to prevent application crash if one frame is lost

### Frame rate control

Since encoding video stream is very CPU intensive, we had to implement a frame rate control mechanism to allow CPU to perform other functions necessary for proper conference behavior (decoding streams from other participants, user events handling, independent applications working e.g. audio tool). A few different approaches to this problem were implemented and tested but so far the best one was the solution based on the decoding time.

After decoding a frame from a conference participant, decoding time is measured. Based on that time, number of participants and experimentally selected coefficient the time between encoding consecutive frames was calculated. (I do not understand this sentence) However this solution works fine only if encoding capabilities of conference participants are similar. If one participant’s is capable of an encoding frame rate much higher than the others, other participants are unable to decode incoming frames. We plan to improve this method by adding to the calculations frame rates of other participants.

Another implemented solution was based on acknowledgments of decoding frame that were send back to the video sender. The sender continued to capture and transmit video only after it obtained acknowledgments from all participants. This solution certainly slowed down the sending process but it had very convenient advantage of auto-regulation. This mechanism worked fine within OpenDVE environment but since NCS application data exchange is based on the raw UDP protocol, some of acknowledgment messages were lost what in turn led to all applications hung up after some time. Obviously, it is possible to implement a more robust protocol, but the associated overhead lowers the overall performance quite significanlty.

### Video format conversions

Both H.261 and H.263 encoders require YUV 411 video format as input. However, nor SGI video capture library neither PC capture library have capabilities to capture video data in this format. SGI format is very similar (YUV 422), so conversion is very simple. PC provides raw RGB format that requires conversion process described in chapter 2 of this report. Such conversion procedures were implemented and successfully used in Video Tool.

## Audio Tool

Audio Tool is application created to enable voice communication between conference participants. It was designed only for the speech signal, however it is possible to transfer also low bandwidth music. Since application was intended for Internet, high sound quality was sacrificed for the bandwidth efficiency. Audio Tool adopts two compression options:

- GSM 06.10 - used in digital cellular telephony, very bandwidth efficient but designed just for the speech transmission.

- Intel/DVI ADPCM - takes more bandwidth but offers much better quality, good enough even for  music.

So far both option exchange 8000 samples per second sound, 16bits sound. Audio stream is coded and sent in the 100 ms samples chunks. Audio formats are automatically recognized and appropriate decompression method is selected.

### Multiplexing

On SGI platforms, multiplexing of audio streams is performed using internal SGI audio library mechanisms. For each stream a dedicated audio port is opened. When receiving audio data, sender ID is detected and audio chunk is decoded and assigned to the appropriate port.

Most PC sound cards do not support multiplexing of data streams. Moreover, usually they support only half-duplex audio mode. To handle this situation Audio Tool implemented on PC uses a completely different approach. First of all, special mechanism switching between sending and receiving mode was created. Initially Audio Tool is in sending mode until the moment when level of audio received from a participant is above some threshold. This threshold can be set manually by user through application graphic interface. After that, application is switched into the receive mode and participant that sent the switching packet is set to the “current sender”. Until next switch to the send mode all audio packets from different participants are ignored. Application is switched back when received audio level drops below threshold for a predefined amount of time.

The process described above avoids necessity of audio multiplexing. It also support a “hands-free” conferencing mode (no “Push to talk” button).

### Buffering

On SGIs, buffering is supported by an internal audio library mechanism. Each audio port has it own samples queue whose length may be set by developer. Playing function blocks until there is enough room for the sample buffer.

Unfortunately, PC platforms do not have such a convenient feature. Buffering is left to the application developer. In Audio Tool the double buffering scheme is applied both for the audio capture and audio playback. Upon reception, the audio chunk is decoded and prepared to playback, but the sample buffer is not sent to the speaker until the next audio packet is received. This next packet is stored in the other buffer while the previous  one is being played. Similarly, one buffer is compressed and sent while the other is being captured.

## Whiteboard

### Overview

Whiteboard is a very convenient collaborative tool that allows sharing of drawings among conference participants. It is possible to perform the following operations:

- drawing lines and basic geometrical shapes

- choosing drawing colors

- choosing line width

- displaying text

- saving and retrieving drawing from a file

### Implementation

Each drawing element is stored in memory as an “graphic object”. Object consists of following information:

- object type

- object position

- additional location

Application creates and manages list of objects that can be also saved and retrieved from a file. Each object type has its own drawing procedure. Objects are exchanged between applications using NCS mechanisms. Objects received are handled the same way as other user input.

So far whiteboard was implemented only on SGI workstations based on Motif library. However, porting to PC platforms is planned, as well as improvement of the application’s clipboard capabilities.

# NCS-Tango integration

Tango is a main NPAC collaboratory system. Tango is more Web-oriented and, being implemented mostly in Java, it does not support real-time applications. Since it was necessary to provide collaboratory environment with functionality of both Tango and NCS these two system were integrated. Because NCS works in this system in slave mode – all the conference management is performed by Tango -- conversion of NCS could be done without knowledge of Tango architecture. Hence, this architecture will no be presented in this report.

To obtain seamless integration of Tango and NCS the following activities were undertaken:

- removal of the existing graphical user interface

- design of a compatible message exchange protocol

- install of a Tango communication unit instead of the NCS GUI

- Figure 5 (a) NCS-user interaction (b) NCS-Tango integration

# Future work

The system created during internship provides basic set of collaboratory mechanisms. The system is far from perfection. Apart from user-feedback and handling unexpected situation, the improvements that are critical include: a more reliable control protocol, video rate control and audio quality. Other projects for the future are:

- implementing multicast support

- adding database support to save and restore conferences from files

- porting the system into multiplatform Java environment and Web

Video on Demand project

Chapter

4

# Overview

Video on Demand (VoD) system allows a user to remotely retrieve content of movie database having full control over transmission process. From the user point of view it works as VCR with capabilities of choosing the movie from a wide programmatic offer.  VoD systems open new possibilities in education, entertainment and business.

# Architecture

A VOD system consists of three major parts:

- Stream server - provides the stream of data from one of the server’s files, allows to start and stop transmission at any time and also to random access the data

- Database - provides the information about the movies available. It includes server address, file path, file length, frame rate, frames offsets and others

- Clients - reads data from the server. It is the client responsibility to process the data from server in proper way based on information from database

VOD system architecture is shown on figure 6.

Communication between database and clients is performed via Netscape Navigator using standard HTTP, CGI mechanisms. Data from the server are transmitted using socket connections (TCP). Transmission control is done by additional control connection based on VOD client-server protocol.

- Figure 6 VOD System architecture

# Audio-Video H.263 Client

AV client is implemented for the SGI Indy, IRIX 5.x platform. It uses H.263 video compression, and either GSM or ADPCM audio compression. It supports QCIF and CIF file formats. It supports interactive movie playback with full random access to the movie stream.

### New elements

In existing movie formats, audio and video data were included in the same interleaved stream. H.263 is a video-only format, and there is no definition of the stream structure. Audio and video are transmitted in two independent streams. Hence it was necessary to open two server connections, send and receive control messages from two separate channels. Unfortunately, database system offered information just about video stream. Therefore, audio configuration had to be included in the data file. It was accomplished by adding special audio header whose structure is presented below in Table 2

- Table 2 Audio header

| Name | Length (in bytes) | Description |
| --- | --- | --- |
| Title | 12 | String “npac-audio”. Indicates that file is NPAC audio stream |
| Rate | 2 | Rate in samples per second |
| Channels | 1 | Channels number: 1 - for mono 2 - for stereo |
| Sample width | 2 | Sample with in bits |
| Code format | 10 | String indicating compression type “adpcm”, “gsm” supported |

### Synchronization

Synchronization is based on the internal SGI Audio Library mechanisms. The procedure of sending audio samples to the speaker port blocks until all previous sent samples are played. In this case it is enough to send audio portion and then decode video frame to achieve the synchronization and keep the frame rate. The only problem is that this requires single video frame decoding, which transfers the responsibility of single frame read-out from the decoder to the AV client.

To obtain this effect, some changes were necessary both at the decoder and client side:

- the original H.263 decoder was transformed into library that allows decoder initialize and then decode single frame passed as function argument

- special function detecting H.263 frame end was implemented and used in single frame read-out procedure

- to make processes of reading data from socket and taking the single H.263 frame independent, a system of ring buffers was implemented

### Random access

Since audio chunks take always the same size, it isn’t difficult to find beginning of a chunk. Situation differs for the  H.263 video stream. First, H.263 frames have different length, second, because it is predictive compression, only INTRA frames can be accessed in this way. The way of solving this problem is described in chapter H.263 stream offsetting.

### Stream offsetting

H.263 system is supposed to be used as a preview tool to the MPEG movies. Hence, H.263 sequence will be acquired by converting MPEG files. For a preview tool it is absolutely necessary to have the capabilities of random access. It implies necessity of offset files generation.

To obtain offset files it was necessary to add some new features to the H.263 encoder:

- possibility of encoding INTRA frame every fixed number of frames (only INTRA frames can be accessed randomly)

- writing offset file with number and stream position of every INTRA frame

- This mechanism allow to generate offset file parallel with movie converting

# Future work

Since H.263 AV client seems very good candidate for preview tool for NPAC VOD system it can be improved in several ways:

- implementing multicast client what enables multi-user video system without delays and high network traffic

- improving client (and server) stability

- creation easy to use MPEG->H.263 conversion tools

Glossary

ADPCM 	Adaptive Differential Pulse Code Modulation

API 			Application Programmers' Interface

CM			Conference Manager

CE			Conference Engine

CIF			Common Intermediate Format

CPU		Central Processing Unit

DCT		Discrete Cosine Transform

FFT			Fast Fourier Transform

GOB		Group of Blocks

GSM		Global System for Mobile Communication

GUI			Graphical User Interface

H.263		one of the latest video-coding standards, emerged from h.261

IP			Internet Protocol

ITU			International Telecommunication Union

IVS			Inria Videoconferencing System

Kbps		kilobit per second

LAN 		Local Area Network

Mbps		megabit per second

MPEG		Moving Pictures Expert Group

NCS		NPAC Conferencing System

NV			Network Video

OpenDVE™	Open Digital Video Everywhere

QCIF		Quarter CIF

RFC		Request For Comments

RLE		Run-Length Encode

RSVP		Resource ReSerVation Protocol

RTCP		RTP Control Protocol

RTP		Real-time Transport Protocol

SD			Session Directory

TCP		Transmission Control Protocol

TTL			Time To Live

UDP		User Datagram Protocol

URL		Unique Remote Locator

VAT		Visual Audio Tool

Vic			Videoconferencing Tool

VLC		Variable Length Coding

WWW		World Wide Web

References

RETTINGER, LEIGH ANNE “Desktop Videoconferencing: Technology and Use for Remote Seminar Delivery” (Under the direction of Dr. Thomas K. Miller III.), 1995, http://www2.ncsu.edu/eos/service/ece/project/succeed_info/larettin/thesis/

Degener, JUTTA “Putting the GSM 06.10 RPE-LTP algorithm to work”, December 1994, http://www.ddj.com/ddj/1994/1994.12/degener.htm

HUNG, ANDY C. “ PVRG-P64 Codec 1.1”, November 1993

DRAFT ITU-T Recommendation H.263, July 1995, http://www.fou.telenor.no/brukere/DVC/h263_wht/

TELENOR “H.263 Advanced Negotiable Options”, December 1995, http://www.fou.telenor.no/brukere/DVC/h263_options.html

BULAWA, JANUSZ “Integration of multimedia collaboratory environment with Web browser” – Master Thesis, September 1996

DRAFT ITU-T Recommendation T.120 “Data Protocols for Multimedia Conferencing”

INSOFT INC. “OpenDVE Architectural Overview”, 1996

VTEL Corporation  “H.320: A Quality Requirement Guide”, 1996