#

# $RCSfile: simle.doc,v $ $Revision: 1.1.1.1 $ $State: Exp $

#

MEMO

TO:     ASD Developers

RE:     Support for SIMNET on SPARC based, SunOS 4.1.1 machines

DATE:   March 6, 1992 - document version 2

FROM:   M. Yeager

There is now support for running SIMNET applications on SPARC based SunOS

4.1.1 machines.  The support consists of:

a simle device driver, with header files

a version of libnetif.a, with header files

the program netxr

a new make.config for Sun

The sources are checked in under source code control in the standard

/nfs/common_src directories (on testament on the u1 disk).  Compiled versions

have been placed on harley in the /common directory (on the u2 disk).  You

must compile on a 4.1.1 machine, 4.0.3 will not work.

DEVICE DRIVER

=============

Please see the file "simle_doc" checked in with the simle driver source files.

LIBNETIF.A

==========

The SPARC version of libnetif.a will support both the older CMC device,

and the newer simle device.  That is, I have not changed the CMC parts of

the library, I have only added code.  You must use the new make.config,

as the old one did not have a particular "define" in the compile line.

(Did the AGPT crew have their own make files?)

An application that does not do any special filtering in the enp.bin

program (which runs on the CMC card) can now choose the device to use

at run time.  Call net_open() or AssocOpen() with either "/dev/enp0" or

"/dev/simle1".  An engineer could use configuration files, command line

arguments, or environment variables to tell the application which device to

open.

An application that does use a special enp.bin to do filtering (for example,

IVIS) will now have to do that filtering in the application itself.  An

engineer could design this code so that it is used or not at runtime

depending on which device is to be accessed.

It may be desirable to bring a little more intelligence to libassoc and have

it decide where certain filtering processing should be executed - either

on the host CPU, or distributed to an intelligent card (CMC).

There is no ringstart or netstart with the simle driver.  Either it is loaded

on the Sun, or it is not.  Use the command "modstat" to check if it is loaded.

Most of the information available from the CMC card is not available through

simle.  Notice that printing the statistics using netcon will show a lot of

"-1"'s.  These are not supported at this time.

Other information is reported in the console window.  The le drivers will

report when the carrier is lost (cable is unhooked), and other error

conditions through output to the console.

Wegman has requested that we provide info on the loss of carrier be available

to a program at run time.  We are talking to Sun about this.

The simle driver supports more than one process concurrently.  That is, you

can run several programs on one machine that send to and receive from the

SIMNET.  Incoming packets are shared and reference counts keep track of

when the packet is no longer needed.  The warning here is that if you run

several programs that subscribe to the same multicast address, all

programs must receive each packet before the space it occupies is available

for reuse.  (If you run casper on a particular exercise in conjunction with

other software running, it had better either receive packets or unsubscribe

and flush the net receive buffers.)

The simle driver does support blocking (the NETBLOCK flag to the various

net_recv() functions), so that programs will not spin while waiting for

a packet.  If you only want to block for a maximum amount of time you

can use the UNIX functions alarm() and signal() to get woken up.

MODES

-----

Several modes of communication are supported:

rebound

no_net

loopback

promiscuous

check_write

These modes are independent of each other, so there are 16 possible

combinations.

To manage these modes include file "simle_ioctl.h".  Below, assume:

int h;

h = net_open("/dev/simle1", 0);

or

h = AssocOpen("/dev/simle1", "some_assoc.def_file");

REBOUND

-------

Rebound means packets sent from one process on a given machine are delivered

to other processes on that same machine.  The packet must pass the

multicast address and type (protocol) filters for the receiving process.  The

default is rebound is enabled.  To keep other processes on the same machine

from seeing packets sent, call:

net_access(h, SIMLESETNOREBOUND);

To return to the default:

net_access(h, SIMLECLEARNOREBOUND);

NO_NET

------

No_net means the packet will not be placed on the network.  Processes on the

same machine can still receive the packet.  The default is no_net is

disabled.  To keep packets off the network, call:

net_access(h, SIMLESETNONET);

To return to the default:

net_access(h, SIMLECLEARNONET);

LOOPBACK

--------

Loopback means that the packets sent by a given process are "rebounded" to that

same process for reception.  There are no multicast or type (protocol) filters

applied, all packets loopback.  The default is loopback is disabled.  For a

given process to see all packets that it has sent, call:

net_access(h, SIMLESETLOOPBACK);

To return to the default:

net_access(h, SIMLECLEARLOOPBACK);

PROMISCUOUS

----------

Promiscuous means that all packets that are received by the LANCE chip are

received by the process.  The catch here is that we do not, at this time,

put the LANCE chip in promiscuous mode.  That is, the hardware still only

receives packets that have destination addresses that are a) the machine's

actual Ethernet address, b) broadcast address, or c) a subscribed multicast

address.  It is possible to set the LANCE chip to receive every packet,

regardless of destination address, but that has not been implemented yet.

(Time ran out.)  If this is important, it could be added.  The default is

promiscuous is disabled.  To receive every packet that the LANCE chip

receives, call:

net_access(h, SIMLESETPROM);

To return to the default:

net_access(h, SIMLECLEARPROM);

CHECK_WRITE

-----------

Check_write means the simle pseudo device driver will wait for the LANCE chip

to indicate transmission status before returning from a write() system call.

If the LANCE cannot properly put the packet on the wire an error will be

returned.  The most common reason for this is the cable is unhooked.   In the

no_net mode there is no packet handed to the LANCE, so write() will always

return success.  The default is check_write is enabeled.  To have the write()

system call return after handing off the packet but not waiting for status,

call:

net_access(h, SIMLESETNOCHECKWRITE);

To return to the default:

net_access(h, SIMLECLEARNOCHECKWRITE);

BUFFER MANAGEMENT

-----------------

Incoming packets are managed differently for programs using libnetif.a

(net_rcv(), etc.) or not (read()).

When not using libnetif.a, the packets are left in the mbufs that SunOS

uses to get the packets from the LANCE.  No fancy buffering is done, only

one process may read() packets at a time.

When using libnetif.a packets are placed in our own simbufs.  These are

tailored to the SIMNET/libnetif point of view.  They include reference counts,

which allow up to 16 processes to share the one Ethernet interface.  Each

process will receive only the packets that pass its multicast address and

type (protocol) filters.  Each process can enable/disable modes independently.

Since the memory that is allocated for the simbufs comes from the kernel

(and is mapped into user space), it is a limited resource.  I wanted to be

careful that it was used efficiently.  Simbufs are set up such that each has

some header information, including pointers to make chains, and space for data.

The simbuf buffers were chosen so that over 95% of the typical SIMNET packets

on the net would fit into one simbuf.  Each simbuf is 512 bytes, the first

simbuf has 452 bytes available for data, any subsequent simbufs in a chain

has 504 bytes for data.  The majority of packets are Vehicle Appearance

packets (around 200 bytes).  Thus, for most network traffic, 512 bytes are used

to hold each packet, as opposed to 1514 bytes in other, CMC, implementations.

The number of these simbufs, collectively called the inqueue, is controllable.

The default is 512, so that 256 kbytes are used.  The inqueue length can

be adjusted by a process if it is the ONLY process running.  All processes

that come latter will find the inqueue already set up and may not adjust it.

The call to change the inqueue length is:

net_resize_inq(h, count);

To find the current length of the buffer the "statistic_2" is used via

net_get_statistics() (or the program netcon).  Note that when using netcon,

even after zeroing the statistics statistic_2 will show 512.  This is because

when netcon runs it does a net_open() which creates an inqueue of the default

length.

You actually get one less simbuf than the inqueue length, because I use the

very first one for bookkeeping purposes.  If the inqueue is 512 long, you

get 511 simbufs to hold packets.

Another detail of buffer management is that libnetif tries to lock the

buffers in memory using the mlock() call.  Unfortunately, this call will only

work if you are super-user.  (Failure has no harmful side effects.)  However,

once the memory is locked, it is locked for everybody.  This means that an

engineer could have a little process that is owned by root and has the

setuid bit set resize the inqueue call net_init_mca() but no net_add_mca()

(so it will not have any packets in the inqueue), and go to sleep.  This would

set up the buffers for any subsequent programs that run on that machine.

(One could view this as a type of netstart.)

Actually, in my testing I have not noticed an improvement in performance when

running as super-user.  Perhaps that is because the packet rates were high, and

the buffers were being accessed frequently, so the SunOS never wanted to

page out the memory that the buffers were in.  This may only matter in

lower packet-rate situations.

As with past device drivers, if the inqueue is full newer packets will be

dropped.  I looked at checking the timestamps and dropping the oldest packets,

but it got complicated since the simbufs are shared.  If could be done, though,

if it was important enough.  It would slow down the receive when the inqueue

was full, maybe to the point of the Sun le software dropping new packets

because the queuing is done during interrupt.

NETWORK.H

---------

We have not changed the macro for NETWORK_DEVICE in libnetif.h.  It is still

"/dev/enp0".  This means programs like netxr or netcon will need to be told

which device to use:

netxr x -d /dev/simle1 ...

DEVICE FILES

============

If you look in the /dev directory you will see a lot more than just "simle1" as

a device file.  You will see a group of 16 files associated with simle1,

simle100 to simle115.  These files are how we get multiple processes to use

the same Ethernet tap.  When net_open("/dev/simle1") is called, it loops

trying to open "/dev/simle100", "/dev/simle101", etc. until it finds a device

that is not in use.  This way each process has a unique entry into the

device driver.  The file "/dev/simle1" is still there because before the

actual open occurs libnetif.a must ask the device what type it is (CMC, 147,

simle) and "/dev/simle1" is used for this purpose.

Note, however, that if you want to use simle in the "raw" state (read() and

write() instead of libnetif), you must open one of these associated files,

and not "/dev/simle1".

You will notice that there is also a "/dev/simle0", and a "/dev/simle2", with

associated files.  This is to accommodate the "one tap solution", and a second

X450 card, respectively.

NETXR

=====

Netxr has changed a bit.  The usage now looks like:

Usage: netxr [switches] {x|r} : x for transmit, r for receive Switches:

-c(ount) repetitions [1]

-m(ulticast) address to subscribe to

-d(evice) network device

-dst(destination) send to [ffffffffffff]

-delay count between sends [0]

-h(elp)

-l(ength) data bytes [1492]

-p(ointer) access packet via pointer

-pr(int) the packet received

-src(source) listen to [any]

-b(locking) on receive

-t(est only) receive NetworkTestPDUs only

-type type_code [21000] - Ethernet type code, 0 is receive any

-w(rites will wait for completion status)

The multicast address is necessary because each subscription is only for that

process, it no longer "sticks".  You cannot use netcon to set an MCA, and later

use netxr to receive it.

The option to check the write completion status allows you to make sure the

packets are getting on the wire.  Not using the "-w" switch allows the sending

netxr to run as fast as possible (as throttled by the delay, below), useful

for doing packet transfer rate measurements.

The delay is necessary because the Sun can try to send packets faster than

they can be put on the Ethernet.  Also, the Sun can send faster than it can

recieve.  The delay is just a "for loop", the count given does not map

directly to a delay time between transmissions.  It depends on the machine's

configuration and load.  Netxr does, however, print the timing information

for the transaction, so one can adjust the delay to get the desired

transmission rate.  Output is:

netxr x -d /dev/simle1 -c 10000 -dst 030102030405 -l 200

..

netxr:  10000 packets transferred

elapsed time =            4153 milleseconds

elapsed time =            4 seconds

milleseconds per packet = 0.415300

microseconds per packet = 415

packets written =           10000

packets per second =         2407

Also, similar timing info is kept for receive, with the clock not starting

until the first packet is received.

Note that on the Sun sending to the broadcast address (all 1's) is much

slower than sending to any other address.  This is because SunOS will send

broadcast packets back to the sender as a kind of automatic loopback.  Then

the simle software has to think about it and discard each packet.

I have made an effort to keep netxr portable to all other platforms. I have

tested it rigorously on the Sun, briefly on the MIPS and Masscomp, not at all

on the GT.  If it is broken, please tell me.

NETCON

======

On the Sun using simle output from netcon looks like this:

netcon -d /dev/simle1 -s

successful transmissions                 0

multiple retries reported                -1

single retries reported                  -1

retry failures reported   (check cable)  -1

deferrals reported                       -1

buffer errors                            0

silo underruns                           -1

late collisions           (check cable)  -1

carrier losses            (check cable)  0

babbling transmitter                     -1

collisions errors                        -1

memory errors                            -1

packets received                         0

missed packets reported                  0

CRC errors reported       (check cable)  -1

framing errors            (check cable)  -1

receive buffer errors                    -1

silo overruns                            -1

stp bit missing                          -1

enp bit missing                          -1

lance errors                             -1

failed type check                        0

failed address check                     0

failed filter check                      -1

statistic_1                              0

statistic_2                              512

length in hdr does not equal pkt length  -1

As you can see many statistics are not reported.  Many of the switches and

commands to netcon will make no sense in the Sun/simle environment.

Successful transmissions means the packet was sent and the LANCE chip reported

success.  There is a layer of software between the simle driver and the

LANCE chip.  This software only reports success or failure, not a reason for

failure.  I have used the carrier losses entry to report these failures.

Buffer errors is a count of mbuf allocation failures.  I have never seen one.

Packets received is a count of packets received, regardless of how many

processes are receiving each one.

Missed packets reported is a count of the number of packets that had to be

dropped because there were no free simbufs.

Failed type and address checks are incremented for each process, and thus

may be larger than the number of packets that actually came in through

the LANCE chip.

Statistic_1 is a performance measure for my own use.  It is a count of ioctl()

calls.  Lower is better.

Statistic_2 is a count of the number of simbufs in the inqueue.

The source code of netcon did not change for the Sun/simle port.

TFX

===

I made an effort to port tfx but ran into trouble.  The problem is one many

of you will find when porting to the Sun, so I mention it here.

The SPARC chip is more fussy on data alignment than the Motorola chips are.

Tfx had a protocol header that had an integer which ended up on an 2-byte

boundary, but not a 4-byte boundary.  This gives a bus error when accessing it.

Just for experiment, I moved the whole data structure over 2 bytes and was

able to move files.

There are 2 ways to work around the problem:

1) Change the protocol so that the most restrictive field in the data

structure will be properly aligned even if the data structure starts

on a 2-byte but not 4-byte or 8-byte boundary.

2) Move offending data into aligned temporary variables using bcopy().

if "this" is incorrectly aligned, but "that" is OK:

bcopy(&this, &that, sizeof(this));

Then use the temp variable for all comparisons.

SHARED LIBS

===========

A bug that I found came up with Sun's shared libraries.  I noticed that in

certain situations using multiple netxr's on a machine that the last one to

exit would complete but have a bus error.  Using the debugger I found that:

exit() called fclose()

fclose() called free()

free() bus errored with a bad address

I had a hunch that it was a shared library problem, and when I linked static

(-Bstatic to cc at link time by setting  LDFLAGS= -Bstatic), it went away.

I cannot imagine why or how fclose() would call free() with a bad address,

but it may happen to you.